
Matthew Denton
Cable Clearinghouse.com

Matthew:

At your request I traveled to inspect 6 reels of 432, loose tube ADSS fiber.

All of the inventory I inspected has been stored outside, and are on wooden reels.  
All the reels are in good condition, and have steel inserts at the center hole for the reel trailer bars. The cable was manufactured by Alcoa Fujikura Limited (AFL). The sheaths have 3 PE jackets and 1 layer of kevlar, with 2 layers of buffer tubes wrapped in a R.O.L. configuration (9 tubes, Blue – Yellow, in the first wrap, 15 tubes, Violet – Dash/Aqua outside) around a P.E coated GRP central strength member. There are good ripcords incorporated within the layers, facilitating preparation. There are 24 buffer tubes of 18 fibers to each tube, the 13th-18th fibers marked approx every 6” with black dots for identification. There is a light flooding (moisture block) inside the sheath.
I have listed below, by sheath number, the date of manufacture, length in feet, and notes pertaining to that reel.

Sheath #
Date of Manufacture

Length

Notes

910032-00

1999


14,422’

Factory protective                                  











wrap on fiber

910031-00

1999


14,184’

No protective 










wrap. Fiber OK

910030-00

1999


14,230’

No protective










wrap. Fiber OK

001116-00

2000


13,608’

Factory protective 










wrap on fiber

001017-00

2000


12,864’

Factory protective










wrap on fiber

001018-00

2000


13,504


Factory protective









wrap on fiber
Please se the pictures attached to this document.  Based on the AFL part number (AN432HIO31CB0), the glass is Corning, SMF-28, single mode fiber, with a max. loss of .35db/km @ 1310nm, and .25db/km @ 1550nm.
Dan Huckans







Matthew Denton


ph:  (404)759-9041





COAXIAL





FIBER OPTICS





ENTERPRISES








DERDAN








