Orbit Reader 20™ User Guide.pdf

Orbit Reader 20 - User_Documentation - # 1319

Orbit Reader 20™ User Guide

Copyright 2016-2018

American Printing House for the Blind, Inc.

Export BRF version

Last Documentation Update: August 31, 2018

Latest Version is .51

Introduction

The Orbit Reader 20 is a portable, refreshable braille display and stand-alone reader with the following features:

a line of 20 braille cells with pins that arrange into the required pattern to accurately represent any six- or eight-dot braille code,

in any language, and for any discipline

reads the contents of files on an SD™ (Secure Digital) card for Stand-Alone mode

remote connectivity to PCs and mobile devices for use with a screen reader

simple editing functions

signage-quality braille

For the latest firmware and software updates, go to Orbit Research Support webpage.

For the most up-to-date documentation, see the Orbit Reader 20 User Guide at tech.aph.org/or20.

To subscribe to the Orbit Reader 20 email list, send a

blank email to or20-subscribe@tech.aph.org.

To participate in beta testing of pre-release firmware, send a blank email to

or20-beta-subscribe@tech.aph.org.

Note: This User Guide addresses the most current version of Orbit Reader 20. If a set of instructions does not work

as expected,

please check the software version and update to the most current version before proceeding.

How the Orbit Reader 20 Is Used

Stand-Alone Mode - Content stored on an SD card is read in one of two ways:

sent by library

prepared on computer and copied to the device

10/24/2018    1/42

In Stand-Alone mode, the Orbit Reader displays the content of files stored on an SD card. It does not do any translation or

interpretation. You prepare the files ahead of time on your computer, then store them on the SD card. For example, if you want to

read in Unified English Braille (UEB), prepare the UEB transcribed title on your computer and then store it on the card.

Remote Mode - Connect the device to a computer or portable device with a screen reader and then do one of the following:

read commercially available titles on Kindle™, Adobe® Digital Editions, or iBooks® application programs

employ any accessible technology for browsing, utilities, and education

The screen reader translates the text into the braille code of choice.

Transcribed Braille

There are multiple sources for files transcribed into high quality braille. Below are some examples.

National Library Service (NLS) provides professionally transcribed, formatted files through the Web Braille project.

American Printing House for the Blind (APH) transcribes textbooks for K-12 students.

National Braille Press (NBP) produces braille books, textbooks, tests, and information for adults and children.

Louis Database lists titles produced by over 160 organizations.

Automatic Translation

In addition to professionally transcribed titles, you may also use dynamic translation to obtain braille.

Automatically translated braille may be appropriate for some forms of reading.

NFB-NEWSLINE® is a free audio information service providing downloadable Braille files of up-to-the-minute content from over

400 national, international, and state newspapers.

Bookshare® dynamically generates braille formatted files from the titles in its library.

Translate Braille

BrailleBlaster™ is a full-featured transcription software package for creating quality formatted braille.

DAISY™ Pipeline provides a comprehensive solution for converting text documents into accessible formats, including braille

PEF (Portable Embosser Format).

Send to Braille is a shortcut that adds braille to the Windows® Send To menu, which can convert files on your computer into

unformatted BRL (Braille Ready Format) files.

Documentation Conventions

For consistency and clarity, this documentation uses the following conventions.

Braille keys are indicated by number. For example, if the documentation indicates Dot 1, it shows: Dot 1.

If multiple braille keys are required, the documentation indicates those keys by showing the numbers separated by a space like this:

Dots 1 4.

When modifier keys are used, the documentation separates the modifiers from the rest of the keys with a plus (+) sign like this:

Space + Dot 1. Recall that modifiers, like the Shift key on a regular QWERTY keyboard, are keys you hold down while pressing

another key. This modifies the effect of the pressed key. On a braille keyboard, Space is often used as a modifier to alter the effect of

the input keys.

Key mnemonics are written in capital letters for emphasis, but are typed in braille lowercase (unless otherwise stated). When one key

follows another, the two keys are shown with a comma between them. For example, the command Select, M means to press and

release the Select key, then press and release M (Dots 1 3 4) simultaneously.

The Up, Down, Right, and Left directional buttons on the navigation pad are interchangeably referred to as arrows, buttons, arrow

buttons, and arrow keys.

All messages from the device are preceded with Dots 2 5, 2 5 and a space. In the documentation, this is indicated by a double

hyphen.

The words Keys and Buttons are used interchangeably.

10/24/2018 2/42

In the descriptions of each Menu option, this documentation indicates the default setting.

Any reference to navigation by "page" while in Stand-Alone mode only means navigation by 1000 characters (page = 1000

characters).

In the Box

Check that all the listed items are in the box you receive. The SD card should already be inserted into the unit. Contact Customer

Service at cs@aph.org or call 800-223-1839 with any questions.

Orbit Reader 20

Standard-A to Micro-B USB cable

AC adapter

SD card with User Guide and content (installed in unit)

Braille Quick-Start Guide

Print Quick-Start Guide

Features

These are a few of the features the Orbit Reader has to offer.

20 refreshable eight-dot braille cells

Eight Braille Input keys and Space bar

Navigation pad with four directional buttons (Up, Down, Left, Right) and the Select button

Micro-B USB charging port

SD card slot

2 Panning rocker keys

Bluetooth® wireless technology

User-replaceable, rechargeable batteries

Orientation

Orientation to the positioning of buttons, keys, cells, and slots is important for understanding how the device operates and how you

input and receive information on the device.

To begin exploration of the Orbit Reader, place the device on a flat surface in front of you with the row of braille cells closest to you.

This is the proper operational orientation.

As you reach around from the front to the back of the device, from left to right, there is the Power button (left), an SD card slot

(middle), and a Micro-B USB port (right). (Image is reversed in photo.) They are all recessed in an area in the left half of the back

edge. The Power button sticks out enough to make it easy to identify. The SD card slot has two small bumps directly below it and the

USB port has one bump below it.

The SD card slot is a standard type with spring feedback. The SD card is inserted with the card connector fingers facing down. To

release the card, press in and remove your finger to allow the card to pop out.

10/24/2018 3/42

The battery compartment is located on the bottom of the device.

Key Placement and Use

When properly oriented, the braille cells are closest to you. The Panning keys are at each end of the braille display. See Panning

Keys section for more

information.

For orientation purposes, there are three slightly raised tick marks located above the braille cells. These orientation marks are

spaced by every fifth braille cell. For example, the first tick mark from the left is between the fifth and sixth braille cell.

Moving toward the top and away from you, find a row of three keys, with a wider one in the middle. The wide key is the Space bar.

The Dot 7 input key is to the left of the Space Bar, and the Dot 8 input key is to the right of the Space Bar.

As you continue toward the top of the face of the display, there is a navigation pad in the middle, between the Braille Input keys and

Space bar.

The navigation pad contains the four directional arrow buttons (Up, Down, Left, Right) and the Select button.

The six traditional braille input keys are aligned horizontally along the top edge of the face of the display, Dots 3 2 1 on the left and

Dots 4 5 6 on the right.

In Stand-Alone mode, the Braille Input keys are used for navigation and editing/writing.

In Remote mode, when using the display with a screen reader on a host device, such as an iPhone, the Braille Input keys can be

used to input text or control functionality of applications.

Panning Keys

The Panning keys are two-function rocker keys found at either end of the braille display. These keys control how you read lines of

text: either next or previous. Pushing on one end of the key results in one action, while pushing on the opposite end results in the

opposite action. To read the next display of text (panning forward), press the bottom part of the key (down) on either Panning key. To

10/24/2018 4/42

read the previous display of text (panning backward), press the top part of the key (up) on either Panning key.

8-Dot Braille Cells

If you are new to refreshable braille displays, you may not be familiar with eight-dot braille. It is similar to six-dot braille with two

additional dots below Dot 3 and Dot 6. These two additional dots are called Dot 7 and Dot 8, and they are used to indicate

capitalization in the Computer Braille Code (Computer Braille), or to indicate some attributed text, or a cursor. Their exact use

depends on the software you use with the display. Check the software documentation for exact usage.

Eight-dot Computer Braille is similar to uncontracted braille, except it features a one-to-one correspondence with the plain text

(ASCII) characters. Computer Braille does not require an additional cell to show a capitalization indicator. Instead, Dot 7 is added to

the lowercase version of the letter. For the ASCII equivalencies in Braille, go to the Appendix A - Computer Braille Chart at the end of

this document.

Getting Started

The Orbit Reader 20 has two modes: a Stand-Alone mode and a Remote mode. In Stand-Alone mode, the device is a book reader

that uses an SD card as the reading media. In the Remote mode, the device works like other braille displays and needs to be

connected to a computer and a screen reader. This section explains how to charge, turn the device on and off, insert the SD card,

and enter and exit the menus. Refer to the following sections for additional support.

Charging the Device

When you receive the Orbit Reader, it may already have a charge; however, APH recommends charging it fully before or during first

use. You can use the device and charge it at the same time, so this does not hinder your reading time.

When the battery in the Orbit Reader gets to 10%, Dot 8 of the 20th cell on the display blinks on and off. This behavior is normal,

indicating that the battery is low.

To charge the device, use the provided Standard-A to Micro-B USB cable and the power plug. Insert the small end of the cable into

the Orbit Reader's USB port with the bumps facing down. It should slide in without resistance; do not force it. Then insert the large

end of the cable into a computer USB connector or the AC adapter provided. Plug the adapter into an AC wall connector.

If the device is on when you connect the charger, the display shows "-- Charger connected" When you unplug the charger, it shows

"-- Charger unplugged" The charger plug and unplug event indications are briefly displayed even when the unit is off. For more

information, see the Battery section under The Menu.

If you have charged the device for 4 or more hours, and it does not power on, contact Customer Service at 800-223-1839 or

cs@aph.org.

Powering On and Off

A square Power (on/off) button is found in a recessed area on the back of the device. See Orientation section for more information.

To power the device on or off, press and hold the Power button until you feel the all the cells flash up and down. This is how the Orbit

Reader lets you know it is coming on or shutting down.

When you turn the device on, Orbit Reader resumes from the last place of the last file in use. If this is the first time you are using the

device, or if this is the first time you are inserting a new SD card, Orbit Reader shows the first file or folder name on the SD card.

If the operating mode of the unit was Bluetooth when last powered off, then the device turns back on in Bluetooth mode.

The Orbit Reader displays, "-- No SD card" when no card is inserted in the SD card slot.

The Orbit Reader 20 has a low-power standby/sleep mode. Quickly tapping the Power button puts the unit in sleep mode. Tapping

the Power button while the device is in sleep mode wakes the device. While editing or when connected to Bluetooth, if no keys are

pressed for 1 hour, the unit automatically goes into sleep mode. Under any other circumstances, the device goes to sleep after 10

minutes of inactivity. After 5 hours in sleep mode, the unit shuts off to conserve power.

Inserting and Formatting the SD Card

The device comes with an inserted SD card with translated braille files already on it, so that you can quickly learn to use the device

by reading the material.

Orbit Reader uses standard full-size SD cards from 4 to 32 GB in capacity. The card must be formatted as Fat32. There is no way to

10/24/2018 5/42

format a card on the device. Most cards come already formatted. However, you can format one as Fat32 on a desktop computer for

use with the Orbit Reader.

To insert the SD card, find the large slot on the back.

On one of the short sides of the SD card there are some ridges called SD fingers or teeth. Situate the SD card with the teeth facing

down. With the device in the correct orientation for use, insert the short side with the teeth into the device. The SD card should go in

smoothly until it gets about a quarter of an inch from being fully in the device. At that point you feel a slight resistance. The card slot

works like a toaster, gently push the SD card in until you hear a click. The card is flush with the rear edge when properly inserted.

To remove the card press it in a little until it pops out (most of the way) and gently remove it.

Note: Removing the SD card while in the Editor, could result in lost work. Be sure to save your work or exit the Editor before

removing the SD card.

About Menus and File Names

Orbit Reader shows internal menus and file names in eight-dot Computer Braille. See 8-Dot Braille Cells section for more information

about eight-dot Computer Braille.

For a complete reference to the characters comprising Computer Braille, see the Appendix A - Computer Braille Chart.

Entering and Exiting Menus

To… Do this

open the menu press Select + Up Arrow

exit the menu press Dot 7

move through the menu choices press the Up or Down Arrows

see choices within a menu option use the Right and Left Arrows

select a menu item press Select

Device Operational Modes

The Orbit Reader 20 has two functional modes. One mode is called Stand-Alone mode. The other called Remote mode, is for use

while connected with another device.

Stand-Alone mode is the default operational mode and allows you to read, edit, and browse files without being connected to another

device. An SD card must be inserted in the Orbit Reader while operating in Stand-Alone mode. See the Stand-Alone Mode section of

this document for more information.

Remote mode allows you to connect the Orbit Reader to iPhone®, iPad®, and Android™ mobile devices, as well as desktop

computers. In Remote mode, Orbit Reader acts as an input and output device for the remotely connected host via USB or Bluetooth.

See the Remote Mode section of this document for more information.

To move back and forth between Stand-Alone and Remote modes, use the following commands:

To switch to Remote mode: press Select + Right Arrow

To switch to Stand-Alone mode: press Select + Left Arrow

Stand-Alone mode operates in the following ways:

Menu

File Manager

Reader

Editor

Remote mode connects in the following ways:

Bluetooth

USB: HID, Serial, or Mass Storage

The Menu

10/24/2018 6/42

Orbit Reader 20 provides a menu for setting options, checking battery, and changing modes of operation.

To open the menu, press Select + Up Arrow. Battery Status is the first menu option displayed. Some menu items let you select

among a number of choices. For example, the Sort item lets you select one of several ways to arrange how files appear in the file

manager. Other menu items provide information about the device, such as serial number and version. The selected option is

underlined with Dots 7 8.

To scroll through the other possibilities, press the Right Arrow key. To select the option, press Select. Orbit Reader responds by

underlining the word with Dots 7 8 to indicate the choice.

To close the menu and return to your work, press Dot 7.

To move to another menu item, press the Up or Down Arrow. When you get to the end of the list of options and press the Down

Arrow again, Orbit Reader moves back to the top menu item. Similarly, when you press the Up Arrow from the first menu item, Orbit

Reader moves to the last item in the list. This feature allows quick access to the last few menu items. That is why some options that

may be frequently referenced are located near the bottom of the menu list.

Menu Availability

The menu is available for the Remote and Stand-Alone modes. Press Select + Up Arrow to open the menu in either mode.

To exit the menu and return to the previous mode, press Dot 7.

Menu Options

A few of the Menu options have a default setting (indicated in the list below). For most of these options, you can move among

alternatives by pressing the Right or Left Arrows and then pressing Select. Selected items are underlined with Dots 7 8. Refer to the

specific section for further information.

Battery

Cursor Blink (1)

Sort (Name: Ascend)

Split Words (Off)

Filter Dot 7 (On)

Wrapping (On)

Compress Spaces (On)

Load Language

Switch Language

Encoding

Version

Serial

Reset Defaults

USB (HID)

Bluetooth (Auto)

Pair (Just Works)

Emulate (Off)

Mode (Stand-Alone)

Battery

The Battery status menu item displays the amount of power (percentage) left in your battery. When the battery in the Orbit Reader

gets to 10%, Dot 8 of the 20th cell on the display blinks on and off. This behavior is normal, indicating that the battery is low.

It also notes when the unit is charging. For example, when plugged in for charging, the message "-- Charging XX%" is displayed,

where XX is the percentage number. Similarly, for first-time use, the message "-- Battery XX%" is displayed.

Cursor Blink (1)

10/24/2018 7/42

The Cursor Blink option only works when editing in the Edit Box or Editor mode. The options set the number of seconds between

blinks. The default setting for the Cursor Blink option is 1, which is one second between blinks. The setting choices are 1, 5, 15, 30,

and 60 seconds, and 0, which sets the cursor to not blink. To make a selection, press Select.

Sort (Name: Ascend)

The Sort menu item lets you change the order of the files on the SD card, using Orbit Reader's built-in File Manager. The categories

are Name (Ascend/Descend), Date, Size, and Last Read. The default setting for Sort is Name: Ascend. To return to the File

Manager, press Dot 7 from Stand-Alone mode. To make a selection, press Select.

You cannot sort in Remote mode, but you can view and change the Menu options. In Remote mode, press Select + Up Arrow. This

command takes you to the Menu. Down Arrow to the Sort list item.

If in the File Manager or the Reader, press Select + Up Arrow to enter the Menu, then Down Arrow to Sort.

If in the Editor, exit out (Select, E OR Select twice) and press Select + Up Arrow to enter the Menu. Down Arrow to Sort.

Additionally, you may sort the files within the selected category in Ascending or Descending order. After selecting a category, press

the Right Arrow until you reach either Ascend or Descend in the list and press the Select button. Orbit Reader arranges the files in

the indicated order. The default setting for this menu item is Ascending (A to Z).

Split Words (Off)

The default setting for Split Words is Off. In the default setting, the Reader tries to put as much content on the display as possible

without splitting a word. However, if you want 20 cells of braille, no matter the content, set Split Words to On. When Split Words is

on, the Reader can show partial words. When you pan to the next 20 cells of braille, the remainder of that word appears at the

beginning of the line. To make a selection, press Select.

Filter Dot 7 (On)

The default setting for Filter Dot 7 is On. The Filter Dot 7 menu item lets you turn off the Dot 7 that frequently appears in some BRL

and BRF files. The Dot 7 is filtered by default, but if you want to make a BRL or BRF file that uses eight-dot braille, turn this setting

off. To make a selection, press Select.

Wrapping (On)

The default setting for Wrapping is On. The Wrapping option is Orbit Reader's way of reflowing files that are already formatted for

embossing. Normally, formatted files contain line-end indicators every 38-40 characters, which starts a new line. This option

eliminates extra spaces, making it easier to read on a 20-cell display. To make a selection press select.

Compress Spaces (On)

Carriage returns, blank lines, and tabs represent extra spaces (or white space) in a document. These spaces take up lots of room

and therefore slow down reading on a braille display. The Compress Spaces option eliminates superfluous blank lines and spaces. If

multiple blank lines or spaces appear in the file, they are compressed to a single space. This setting is especially useful when

reading formatted text files that may contain many blank lines.

The default setting for Compress Spaces is On. To make a selection, press Select.

Load Language

The Load Language menu option is used to upload language files to the flash memory of the Orbit Reader 20 device. The default

translation table for Orbit Reader 20 is English Braille ASCII, which comes preloaded on the device.

The Load Language option has a sub menu of three options: Load Locale, Load .loc Only, and Load .lan Only. To navigate and

select sub menu options:

1. Press the Right or Left Arrow keys to navigate the sub menu.

2. Press Select on the desired item.

3. Press Select again, to select the option.

Load Locale - Orbit Reader displays a region-code-name list for the languages. The list has both the Table files (.lan) and System

Messages files (.loc) in the Locale folder on the SD card, along with the default option. Press Select for the file of your choosing.

Load .loc Only - Orbit Reader displays the list of System Messages files, along with the default option. Press Select for the file of your

choosing.

Note: You need to ensure that the correct ASCII to Braille table of the desired region has been uploaded.

Load .lan Only - Orbit Reader displays the list of Table files, along with the default option. Press Select for the file of your choosing.

10/24/2018 8/42

See the section Upload the Localization Files under the section Language Options (Localization) for additional information.

Switch Language

The Switch Language menu option is used to change between a loaded language and the default English language.

Note: You must first load a language to the flash memory of the device, before using the Switch Language option.

See the Language Options (Localization) section for more information.

Encoding

Text files are created in either ASCII or Unicode encoding format. Text files for some of the languages are created in Unicode only.

When creating a file, in other than ASCII-based languages, you must select the Unicode option. Press Select to choose the option.

ASCII is the default option. You can choose from the following encoding formats for the new file to be created on the system:

ASCII

UNICODE16LE

UNICODE16BE

UTF-8

If you want to create BRF/BRL files, choose the ASCII option. Otherwise, the file might be unreadable by other programs.

This setting is only applied to the files created using the “create new” command. It has no impact on the existing files. The format

cannot be changed while a file is open for editing.

Version

The Version menu item displays the version of the software running on your device. You may need the version number when talking

with Customer Service. For the latest version update, see Orbit Research Support webpage.

Serial

The Serial Number menu item shows the individualized serial number of the device you are using. This number may be important for

warranty purposes.

Reset Defaults

The Reset Defaults menu item resets all your settings to the factory settings. To reset to default settings, press the Select button.

This action resets defaults and takes you to the top of the Menu Options list (Battery Status).

USB (HID)

The USB menu item selects between the three possible USB protocols: Human Interface Device (HID), Serial, or Mass Storage. The

default setting for USB is HID. To select the option, press Select.

For more information, see the USB Connectivity section under Remote Mode.

Bluetooth (Auto)

There are three choices under this menu option.

1. Bluetooth Auto

2. Bluetooth Manual

3. Bluetooth OFF

The default setting for Bluetooth is Auto. If there is any screen reader activity running on the paired device, the Orbit Reader 20

automatically switches to Bluetooth Remote mode. If you do not want automatic Bluetooth connection, select the Manual option. To

disable Bluetooth completely, select OFF. To select another option, press Select.

For more information, see the Using the Bluetooth Connection section under Remote Mode.

Pair (Just Works)

The Pair menu item selects a Bluetooth pairing scheme. Orbit Reader uses the selected scheme when it responds to a Bluetooth

pairing request from a host device. To select the option, press Select.

10/24/2018 9/42

The two choices are:

1. Just Works - the default Bluetooth pairing scheme. When using this scheme, Orbit Reader automatically pairs to any device that

sends the pair request.

2. Confirm code - is the most secure pairing scheme. With this scheme, Orbit Reader responds to pairing requests by showing a

random number on both the Orbit Reader's braille display and on the host device. To confirm the request, ensure the numbers

match and press Dot 8 on the Orbit Reader.

Emulate (Off)

The Emulation option allows you to choose how the device appears to the Host screen reader applications while operating in Remote

mode. If RB18 (Refreshabraille 18™) Emulation mode is selected, the Orbit Reader appears as a Refreshabraille 18 and only the

first 18 cells are used. If Emulation mode is Off, it appears as Orbit Reader 20. The default setting for Emulation mode is Off. For

more information, see the Using the Bluetooth Connection section under Remote Mode. To select the option, press Select.

Mode (Stand-Alone)

By default, Orbit Reader operates in Stand-Alone mode where you read and write files stored on the SD card. To specifically set the

unit to another mode, select among Stand-Alone, BT (for Bluetooth,) or USB. Orbit Reader tries to automatically switch to Bluetooth

or USB depending on screen reader activity, but if you want to, for instance, specifically switch back to Stand-Alone from Remote

mode, or switch the interface from Bluetooth to USB, use this option. To select the option, press Select.

Stand-Alone Mode

Stand-Alone mode is the default operational mode and allows you to read, edit, and browse files without being connected to another

device. A formatted SD card must be inserted in the Orbit Reader while operating in Stand-Alone mode. That card must contain the

files you wish to read.

Stand-Alone mode operates in the following ways:

File Manager

Menu

Reader

Editor

File Manager

File Manager is where you select a file stored on the SD card to read on the Orbit Reader. You can also create new files and folders,

view the properties (such as the file size and current reading position), and copy, rename, and delete files.

When moving through the list of files, Orbit Reader shows the first 20 characters of the file name. You may scroll to the rest of the

information about the file, such as the rest of a long file name, its size, and date, by using the Panning keys to move the display

window through the rest of the contents on that line of the list of file names.

Each item on the file list includes the following items:

File name and extension

Current position in the file in number of characters

Size in KB (kilobytes)

Last modified date

Protected or Unprotected

Using the APH History file as an example, the line should look like this:

APH History.brf (File Name and extension) Pos: 0 Size: 259KB Date: 11-12-2015 Unprotected

Only one column is displayed per line (with panning as necessary). Pressing Left or Right Arrow moves to the previous or next

column heading (file information). Upon exiting the Reader (Dot 7), you are returned to the same place and column in the File

Manager.

File Manager Commands

Following are commands you can use in the File Manager. Most of the commands for files also work for folders.

10/24/2018 10/42

To... Press Additional information

Open a file or folder Select or Dot 8 Pressing Select or Dot 8 opens the file or

folder.

Go to previous or next file or folder Up or Down Arrow Pressing Up or Down Arrow moves to the

previous or next file in the list.

Go back one folder level Dot 7 Dot 7 acts like a "back" key to exit the

current activity. If you are already in the

File Manager, pressing Dot 7 moves the

focus back one level when you have

multiple subfolders open.

If you are at the root level, Dot 7 does

nothing. If you are in one of the columns

of information, for example size or date,

then pressing Dot 7 returns to the

filename.

Move through file information Left or Right Arrow This command moves you through the file

information by headings. See File

Manager section for more information.

Open the Menu Select + Up Arrow Pressing Select + Up Arrow opens the

menu.

Scroll text Left or Right Panning keys If the file name is longer than the display,

panning left or right displays more of the

file name. When you get to the end of the

file name, pressing the panning key

moves you to the next item of file

information, such as position, size, date,

and so on.

Make a New File Space + N (Dots 1 3 4 5) The New File command starts the Editor

mode with a new file in which to type. To

exit the Editor, press Select twice.

Make a New Folder Space + O (Dots 1 3 5) The New Folder command opens the Edit

Box where you may type the name of the

new folder. Pressing Dot 8 saves the

folder name and returns you to the file list

with that folder selected. Press Select to

exit without making a new folder.

Delete a file or folder Space + D (Dots 1 4 5) This action deletes the currently selected

file(s) or folder(s). Only empty folders can

be deleted. A confirmation is required

before the file or folder is deleted. When

the message "-- Confirm action" is

displayed, press Select or Dot 8 to

confirm; press Dot 7 to cancel.

Rename a file or folder Space + R (Dots 1 2 3 5) The Rename File command allows you to

change the name of a file or folder. To use

it, press the Rename command. Orbit

Reader responds with an Edit Box

containing the original file or folder name.

Edit the name or type a new one and

press Dot 8. Press Select to exit without

renaming a file or folder.

Cut a file Space + X (Dots 1 3 4 6) This action cuts the currently selected

file(s) from their location and places

it(them) on the clipboard to be used with

the paste command.

Copy a file Space + C (Dots 1 4) The Copy File command makes a copy of

the currently selected file(s) and places

it(them) on the clipboard to be used with

the paste command.

Paste a file Space + V (Dots 1 2 3 6) This action pastes the file(s) or folder(s)

10/24/2018 11/42

that are currently on the clipboard into the

current location in the File Manager. If no

file or folder is on the clipboard, no action

is taken.

Mark a file Space + M (Dots 1 3 4) When moving multiple files, use the Mark

File command to mark each individual file

before using Cut, Copy, or Delete. To

Mark a file, arrow to it and press Space +

M.

To clear a marked file, press Space + M

again.

File Manager indicates marked files by

raising Dots 7 and 8 of the first letter of

the file name.

Protect or unprotect a file Space + P (Dots 1 2 3 4) To protect a file from accidental changes,

arrow to the file and press Space + P. To

remove the protected status, select the file

and press Space + P again. To determine

if a file is Protected, arrow Right or Left to

the Unprotected/Protected heading.

Edit the current file Space + E (Dots 1 5) When editing the current file, the Orbit

Reader switches to an Edit mode, which is

indicated by a cursor at the point of input.

The cursor is at the first character shown

on the display when you pressed the edit

command. As you type, everything is

saved to the file. To exit, press Select

twice. See The Editor section for more

information.

Edit Last Edited File Space + Dots 1 2 4 6 Reopens the last file you edited and

returns the cursor to its last position in that

file. This command is useful for taking

notes on what you read. Use the

command to quickly switch back to the

Editor. Press Select twice to close the

Editor and return to the title you are

reading. This command has no effect if

you are already editing a file. The device

displays the error message "-- File not

found" when no previous file has been

opened or if the file was deleted.

More About Copying and Pasting Files

When copying or pasting a file that results in another file with the same name, File Manager adds an extension to indicate the new

file is a copy. For example: if you copy book.brf to a folder that already contains book.brf, the new file becomes book1.brf and the

original book.brf remains unchanged.

If the SD card runs out of space before a file operation completes, File Manager shows, "-- Error copying file xxx" where xxx is the

file's name.

When copying large numbers of files, or large files that can take some time, File Manager displays progress messages during the

copy operation.

If copying a file that takes more than five seconds, File Manager displays, "-- Copying xxx (yy%)" where xxx is the name of the file

and yy is the percentage complete. This percentage is updated at least every 10% of the progress.

For the command keys, go to the File Manager Commands table under the File Manager section.

File Movement Commands

To... Press Additional information

Move to first file Press Space + Dots 1 2 3 Pressing Space + Dots 1 2 3 moves the

File Manager selection to the first file in

10/24/2018 12/42

the current folder.

Move to last file Press Space + Dots 4 5 6 Pressing Space + Dots 4 5 6 moves the

File Manager selection to the last file in

the current folder.

Incrementally search file list Type the file name quickly To find a file, quickly type the first few

letters of the file's name. File Manager

highlights the first file matching the letters

you typed.

File Types

Orbit Reader does not translate or interpret any file. It simply displays the contents of a file in braille. So, if you put a plain text file

onto the SD card, Orbit Reader displays that file as it was written. Similarly, if you put a BRF file on the SD card, no matter the

language or code, Orbit Reader accurately shows its contents. To use Word or PDF files, you must first translate them on a desktop

computer, then send the translated file to the SD card.

Braille files may contain any kind of braille, either six- or eight-dot, in any language and for any topic. You create these files on a

computer with a translation software package, such as Duxbury or LibLouis, or you create them in the Editor on the Orbit Reader.

Supported File Types

Extension Type Explanation

BRF Translated, formatted braille displays exact representation

BRL Translated, unformatted braille displays all text

TXT Text displays eight-dot Computer Braille

In addition to the supported file types, Orbit Reader shows the contents of any file. Be aware, however, that some files do not contain

readable content. This feature is useful, however, to examine files that do contain readable text even if it is not a supported file type.

There are several excellent braille translation systems available in the U.S. If you obtain textbooks from APH or get transcribed titles

from NLS or NBP, the braille should be correct and formatted. You may also get braille translation software that produces

automatically translated braille using programs like Liblouis or have Bookshare automatically generate braille files of the titles

available.

Preparing Files

To prepare files for placement on the Orbit Reader, follow these guidelines:

Determine the translation type

Translate the file

Place the file onto the SD card or use the Mass Storage protocol to copy from your desktop computer

To copy files to the SD card, remove the card from the Orbit Reader and place into a card reader on your computer, or connect the

USB cable from the Orbit Reader to your computer and activate the Mass Storage protocol on the Orbit Reader by either pressing

Space + Dots 5 7 or selecting Mass Storage from the USB option in the menu. When using the Mass Storage protocol, Orbit Reader

shows up as a drive on the computer, and you cannot use other functions of the Orbit Reader.

The Reader

In Stand-Alone mode, the Orbit Reader displays the content of files stored on an SD card. It does not do any translation or

interpretation.

The Orbit Reader 20 is designed to be a stand-alone reader. Placing BRF, BRL, or TXT content (see File Types section) on an

inserted SD card turns it into an on-the-go braille book reader. When first powered on, the Orbit Reader displays the contents of the

first file on the SD card. To get to the file list, press Dot 7. To select a file, Up or Down Arrow to the file. Press Dot 8 or Select to open

the file. Use the Left and Right Panning keys to read it. There are many other functions in reading mode such as Find, Power Move,

Bookmarks, and even a menu to set preferences. The following sections describe commands and settings that can be used while

reading.

When you turn the device on, Orbit Reader resumes from the last place of the last file in use. For first-time use, it displays the first file

or folder on the SD card. Press Dot 7 to close the file and display the File Manager, which shows a list of the files on the SD card. If

there is no SD card inserted, the message "-- No SD card" is displayed.

10/24/2018 13/42

When you reach the end of a file, "-- End of file" is displayed by the Reader. Similarly, if you are at the beginning of the file, "-- Start

of file" is displayed.

Reader Commands

While in Stand-Alone mode, the following commands are available.

To... Press Additional information

Go to Previous or Next display Panning keys See Panning Keys section for more

information.

Go to Previous or Next Character Left or Right Arrows If you press the Right Arrow, the next

letter appears on the right side of the

display.

To conform to traditional braille display

navigation commands, the Reader also

supports Space + Dot 3 and Space + Dot

6.

Go to Previous or Next Word Space + Left or Right Arrows If you press Space + Right Arrow, the next

word appears on the right side of the

display. The Reader also supports Space

+ Dot 2 and Space + Dot 5.

Go to Previous or Next Line Up or Down Arrows Pressing the Previous or Next Line

command moves to the previous or next

carriage return in the file. Also works with

Space + Dot 1 and Space + Dot 4.

Go to Previous or Next Page Dot 3 or Dot 6 Pressing Dot 3 or Dot 6 moves the cursor

back or forward in the file to the next page

marker (form feed) or 1000 characters,

whichever comes first.

Go to Top or Bottom of a file Dot 1 or Dot 4 Pressing Dot 1 moves to the top of a file;

pressing Dot 4 moves to the bottom of a

file. (For compatibility with other devices,

Space + Dots 1 2 3 moves to the top and

Space + Dots 4 5 6 moves to the bottom

of a file.)

Exit File Dot 7 If reading a file, pressing Dot 7 returns

you to your previous location.

Open the Menu Select + Up Arrow Pressing the Select + Up Arrow opens the

Menu.

Set or Clear a Bookmark Space + M In addition to the automatic bookmark

placed when you close a file, Orbit Reader

lets you set additional bookmarks. Reader

displays the message "-- Bookmark

added" or "-- Bookmark cleared" if there

was already a bookmark at that position.

Go to Previous or Next Bookmark Dot 2 or Dot 5 The Reader responds by moving the

cursor to the bookmarked position. The

bookmarked location appears as the first

character on the display. If there are no

marks in the indicated direction, "-- No

more bookmarks" is displayed.

Power Move/Search Forward and/or Back Dot 8 + Down Arrow or Dot 8 + Up Arrow Press Dot 8 + Up Arrow to move back.

Press Dot 8 + Down Arrow to move

forward. See Power Move Forward and

Back section for more information.

Find Braille Space + F (Dots 1 2 4) Pressing Space + F opens a blank input

field (Edit Box) with a cursor. This allows

you to perform a search in a specific

braille format by typing the text in the

same braille format. Press Dot 8 to begin

10/24/2018 14/42

the search. To exit the Edit Box without

performing a search, press Select. See

Edit Box and Find Braille sections for

more information.

Find Next Braille Dot 8 + Right Arrow Pressing Dot 8 + Right Arrow repeats a

previous search forward from the point in

the file where you are. If there is no text in

the Find Edit buffer, the Reader uses the

text at the cursor. See the Find Braille

section for more information.

Find Previous Braille Dot 8 + Left Arrow Pressing Dot 8 + Left Arrow repeats a

previous search backwards from the point

in the file where you are. If there is no text

in the Find Edit buffer, the Reader uses

the text at the cursor. See the Find Braille

section for more information.

Edit Current File Space + E Space + E to edit the file you are currently

reading. Press Select twice to exit the

Editor and continue reading.

Open New File Space + N Space + N opens a new file in the Editor

for editing/note taking. Press Select twice

to return to the file you were reading.

Edit Last Edited File Space + Dots 1 2 4 6 Using this command reopens the last file

you edited and returns the cursor to its

last position in that file. The Edit Last

Edited File command is useful for taking

notes on what you read. Use the

command to quickly switch back to the

Editor. Then close the Editor to return to

the title you are reading. This command

has no effect if you are already editing a

file. The device displays the error

message "-- File not found" when no

previous file has been opened. Press

Select twice to return to the file you were

reading.

Power Move Forward and Back

Orbit Reader's power move commands provide a quick means of moving/searching through large sections of a file to find a known

location in the document. A power move operates by using boundaries and moving 50% between them with each press of the key

command. In essence, by using a power move to search within a file, you are drilling down to the information using a binary

command. When you press Dot 8 + Down Arrow or Dot 8 + Up Arrow, the Reader moves in the following ways:

On the first press, the cursor moves half way between the current position and the start (Dots 8 Up Arrow) or end (Dots 8 Down

Arrow) of the file, which act as the boundaries for the power search.

Subsequent presses treat the moved-from position as the new boundary in that direction. So, if you are at the beginning of the file (or

0%) when you press the Dot 8 + Down Arrow, the Reader moves the cursor to half way between 0% and 100%. You are now at 50%,

and it becomes one of the boundaries. The initial boundary is at the original position of 0%. Therefore, if you press Dot 8 + Up Arrow,

you move half way between 50% and 0% to end at 25%. Or, if you press the Dot 8 + Down Arrow, you move half way between 50%

and 100% to end at 75%.

Each time you press contiguous Dot 8 + Up Arrow or Dot 8 + Down Arrow keys, the Reader shows the percentage in the first few

cells of the display line and fills the rest with the text at that position. This continues until you press something other than Dot 8 + Up

Arrow or Dot 8 + Down Arrow or a Panning key. See Panning Keys section for more information.

Find Braille

To find specific braille text, use the Find command (Space + F). Orbit Reader responds by opening a blank input field (see Edit Box

section) with a cursor,

which is indicated by Dots 7 and 8. Unlike the Power Move, which searches by known location of text, the Find

command searches the entire file for instances of the braille text you enter in the input field. If you had searched for something

previously, that text appears in the Edit Box with your cursor at the end. See the Power Move Forward and Back and Edit Box

10/24/2018 15/42

sections for more information.

To find a word, press Space + F to enter the Edit Box. Type the braille text you wish to find, then press Dot 8 to start the search

moving forward in the file. To search moving backward in the file, press Dot 8 + Left arrow.

The word is shown as the first word on the display. If the word is not found, the message "-- Not found" is displayed. Press Dot 7 to

clear the message. To exit the Edit Box without performing a search, press Select.

After the word is found, to continue searching, press Dot 8 + Right arrow to find the next occurrence (forward in the file) or Dot 8 +

Left arrow to find a previous occurrence (towards the beginning of the file).

If you are searching for text in a contracted braille file, you must type the text you want to search for in contracted braille.

If you are searching in a plain text file, you must type the text you want to search for in Computer Braille. See Appendix A - Computer

Braille Chart for

the Computer Braille Chart.

You can search for terms up to 255 characters long. Searches are generally not case sensitive. A search for the word "animal"

results in finding both lowercase and uppercase versions of that word (animal/Animal). However, if you want to find words with upper

case letters specifically, type the text you wish to find with the braille indicator for capitalization included. (In contracted braille, one

dot 6 precedes a capital letter while two dot 6s precede an entirely capitalized word. In Computer Braille, capital letters include dot 7.)

For example, a search for the capitalized word "Animal" gives the results for "Animal" (capitalized), but not "animal" (lowercase).

Additionally, searches match both whole and partial words. Searching for "an" finds "an" (whole) and "man" (partial). To restrict

searches to whole words, add a space at the beginning and end of the word. The Reader matches the space with any non-printing

character such as spaces, tabs, and line feeds.

Read the section on Edit Box for editing commands and navigation functionality.

When you search files, the Reader moves the file reading position to the next occurrence of the searched for text and displays it at

the beginning of the display line. If it cannot find the text, the Orbit Reader displays "-- Not found" and leaves your file reading

position where it was before the search. If searching through a larger document, the Orbit Reader displays "-- Busy" while conducting

the search.

To repeat a search moving forward toward the end of the file, press Dot 8 plus Right Arrow.

To repeat the search moving back toward the beginning of the file, press Dot 8 + Left Arrow.

Edit Box

Orbit Reader presents a text dialog or input field called an Edit Box when you want to find text or rename a file. The Edit Box permits

only one line of text. Press Space + F to begin a search or press Space + R to rename a file.

Once in the Edit Box, insert text by typing the desired braille text and then use the Edit Box commands to navigate. The Edit Box

displays a blinking cursor (Dots 7 8) to indicate where you input the text you are searching for or editing.

When Orbit Reader shows an edit box, it often provides default text, such as a file name or a previously entered search term. The

entire text is selected (as indicated by the Dots 7 8 combination added to each letter.) To delete or replace the selected text, type a

letter or press the backspace key (Dot 7). To edit the existing text in insert mode, press one of the arrow keys to move the cursor

letter by letter to the position where you wish to edit.

Edit Box Commands

To... Press Additional information

Move Left or Right One Letter Left or Right Arrow Pressing the cursor movement commands

moves the cursor in the indicated

direction. When the cursor reaches the

end of the edit box, it stops--it can move

no further.

Move to Start or End of Text Up or Down Arrow Press Up or Down arrow to move the

cursor to the beginning or end of the text.

Backspace Dot 7 The Backspace key Dot 7, deletes the

letter immediately left of the cursor. Use

Backspace when you make a typing

mistake or to remove small amounts of

text.

10/24/2018 16/42

Accept typed text Dot 8 In this instance, pressing Dot 8 acts as an

Enter/OK key.

Close the Edit Box Select To close the edit box, use the Close

command. In single line controls, Close

throws away what you typed. In multiple

line controls, all text is automatically

saved in the Editor.

The Editor

In addition to serving as a tool for reading braille, Orbit Reader offers the ability to write and edit text. This feature allows you to

create/edit new or existing files on the SD card.

While you are in the Editor mode, the cursor notes the current position by blinking Dots 7 8. The default setting for the cursor is 1

second. The blinking of the cursor can be turned off from the Menu. See the Cursor Blink (1) section for more information.

There are three ways to edit a file. Each of these three commands is available both from the Reader and the File Manager.

1. Edit new file: Space + N

2. Edit current file: Space + E

3. Switch to last edited file: Space + Dots 1 2 4 6

To insert text, type or paste the text at the cursor. The Editor adds the typed or pasted text. You can insert any text into an open file

using the braille keys (Dot 1 to Dot 8).

To exit the Editor, press Select, E or Select twice. The cursor disappears, returning you to the Reader in the file you were editing.

If you are exiting from a new file, the editor automatically names the file with the first 20 characters in the file. If a file already exists

with that name, the Editor appends a number to it until it produces a unique name. To rename a file or to add an extension to the file

name, highlight the file name in File Manager, and use the rename command. If there are Computer Braille characters that cannot be

used in a filename, such as a forward slash (/) or an asterisk (*), the characters are replaced with an underscore (_). Once you exit,

the new file name in the File Manager is selected. If you wish to rename the file, use the rename command (Space + R). If you want

to give the file a specific name, write that name as the first line in the file.

Note: It is recommended that you exit the Editor before you perform a system update, reset the device, or remove the SD card.

Exiting the Editor saves your work. Not exiting the Editor first could result in lost work.

Editor Commands

To... Press Additional information

Edit New File Space + N Creates a new file in the Editor, ready for

input.

Edit Current File Space + E Opens the selected file for editing.

Switch to Last Edited File Space + Dots 1 2 4 6 Opens the last edited file for additional

editing. The device displays the error

message "-- File not found" if the file has

been deleted.

Add a new line (Enter) Dot 8 Pressing Dot 8 adds a new line (enters a

carriage return) at the end of a paragraph

and places the editing cursor in the first

cell of the next paragraph.

Delete a character Dot 7 Pressing Dot 7 deletes the character to

the left of the editing cursor.

Exit Editor Select twice OR Select, E When you close a file, Editor saves and

returns back to the Reader or File

Manager, depending on your starting

location. It automatically saves your work.

If you opened the file from File Manager,

File Manager makes the new file the

current item in the list.

If you edited an existing file, Reader

moves the reading position to the place

10/24/2018 17/42

where the cursor was.

Save current work Select, S The Orbit Reader automatically saves files

every 1 kB of data or about 1000

characters, as you edit. However, small

file increments could be lost during certain

situations such as, SD card removal,

system updates, or device resets.

Pressing Select, S saves the file as you

deem necessary.

Note: It is recommended that you exit the

Editor before removal of the SD card,

system updates, or device resets.

Go to next or previous character Left or Right Arrow Pressing Left or Right Arrow moves the

cursor to the previous or next character.

Go to next or previous word Space + Left or Right Arrow Pressing Space + Left or Right Arrow

moves the cursor to the start of the

previous or next grouping of letters and

digits.

Previous or next display Pan up or down Pressing up or down on the Panning

rocker keys allows you to move to the

previous or next display of braille. If you

press the bottom part of the rocker key,

the display pans forward. If you press the

top of the rocker key, the display pans

backward. The editing cursor is placed at

the first cell. See Panning Keys section for

more information.

Previous or next paragraph Up or Down Arrow Pressing the Up or Down Arrow moves

the editing cursor to the start of the

previous or next paragraph. If the start of

the previous or next paragraph is not on

the display the display pans the start of

the previous or next paragraph to the first

cell on the display. The editing cursor is

placed at the first cell.

Start or end of paragraph Space + Dot 7 + Left or Right Arrow Pressing Space + Dot 7 + Left or Right

Arrow moves the cursor to the start or end

of the current paragraph. If the start or

end is not on the current display the

display pans the start or end to the first

cell of the display. The editing cursor is

placed at the first cell.

Previous or next page Space + Dot 7 + Up or Down Arrow Pressing Space + Dot 7 + Up or Down

Arrow moves the cursor to the previous or

next page (1000 characters or form feed

character). The first character of the page,

whether previous or next page, is

displayed in the first cell of the braille

display.

Go to top or end of document Long press Up or Down Arrow Long pressing Up or Down Arrow moves

the cursor to the top or bottom of the

document.

Editor Block Text Commands

The Editor block operations depend on a selection. The selection is the text between the cursor and the location mark.

Note: The size limit for a cut/copy/paste operation is 5 KB of data.

Adding a location mark does not change the content. Marks are used for copying/cutting a selection of text. To copy or cut using

marks, press Select, M to place a mark. Only one mark can be placed/added at a time. Once the mark is placed, the text between

the cursor and the mark can be copied (Select, C) or cut (Select, X). Once text is copied or cut, the mark is cleared.

10/24/2018 18/42

To... Press Additional information

Set or clear Location Mark Select, M The editor indicates a mark by raising

both Dots 7 and 8 on the character where

the mark occurs. To set or clear a mark in

the file, press the Mark command. If there

was already a mark at the cursor, the

editor shows "-- Mark cleared" on the

display and removes the mark. If there

was not a mark, it shows "-- Mark set" and

adds a mark.

Copy Select, C The copy command places the selected

text onto the clipboard for later use in this

or another file.

Cut Select, X To remove the selected text from the

document and place it onto the clipboard,

use the Cut command. If you want to put

that text somewhere else, move to the

desired position, then use the Paste

command.

Paste Select, V To paste the contents of the clipboard

before the cursor, use the Paste

command.

Context Menu

The Context menu is is a discoverable method to find commands for use in the Editor. It gives you quick access to functions such as

cut, copy, and mark.

While in the Editor, press Select to open the Context menu. There are two ways to make a selection from the Context menu after it is

open.

1. If you do not know the braille shortcut, press the Up or Down Arrow key to scroll to your selection and press Select.

2. If you know the braille shortcut for the command (the braille letter associated with the function), press that letter.

Braille Shortcut Function

E Exit

M Mark

C Copy

V Paste

X Cut

F Find

S Save

You can also press Select twice to quickly exit the Editor. The first press opens the Context menu; the second press selects the first

option in the menu, which is Exit.

Remote Mode

In addition to using Orbit Reader as a portable reading tool and editor, it connects with host devices (i.e., computers, phones, tablets,

and Orion TI-84 Talking Graphing Calculator) to provide braille input/output to that device. The host device must be running software

that supports braille.

Host Device Screen Reader

Windows PCs JAWS®, Window-Eyes, System Access, NVDA, Dolphin

ScreenReader™

Mac® computers and iOS® devices VoiceOver

10/24/2018 19/42

Android devices BrailleBack, Braille TTY

Chromebook ChromeVox

When you use Orbit Reader as a display for other hosts, the screen reader on that host device provides the translation and other

braille settings. Consult the documentation for your screen reader.

The only hotkeys used with Remote mode that are not sent to the remote device are:

Pressing Select + Up Arrow opens the Orbit Reader Menus.

Pressing Select + Left Arrow returns Orbit Reader to Stand-Alone mode.

Pressing Select + Right Arrow returns Orbit Reader to Remote mode.

Pressing Select + Down Arrow brings up an Edit box where you can type and submit up to 255 characters of text to the remote

device. Press Dot 8 to submit or Select to cancel. See the Buffering Text Input to Remote Devices section for more information.

Before you Connect

Before you connect Orbit Reader, it is important to set the device so that the screen reader(s) you use recognize it. Newer versions

of screen readers recognize the Orbit Reader. The table below shows the first version of the screen reader to directly support Orbit

Reader.

Screen Reader Version

VoiceOver on iOS 10.2

VoiceOver on Mac 10.11

NVDA 2017.1

ChromeVox 61.0.3163.120

System Access 3.7.85

Dolphin ScreenReader 16.05

If you have an earlier version of a screen reader, or one that does not yet support Orbit Reader, you must set Orbit Reader to

emulate Refreshabraille 18.

Note: In Refreshabraille 18 emulation, only the first 18 braille cells are used.

To set the device to emulate Refreshabraille 18, select Emulation from the Orbit Reader menu and select RB18. If you plan to

connect multiple devices and any of your preferred screen readers do not yet support Orbit Reader, you must use the same

emulation setting for each. Note that if you already have emulation set and you want to change the setting, you must disconnect the

device on the host first. You may not alter the emulation setting while actively connected to a screen reader. Temporarily turn off your

screen reader's braille support before altering the setting.

If you already paired Orbit Reader with Emulation off, unpair it from any Bluetooth connections before using it again.

Using the Bluetooth Connection

Bluetooth is a technology that wirelessly connects devices, such as the Orbit Reader 20, to host devices, such as phones, tablets,

and computers. For example, when using an iPhone with VoiceOver, you can control the iPhone with the keys and buttons on the

Orbit Reader, and you can read the entire interface in braille as you interact with it.

If the Orbit Reader is on when you wake up or turn on the host device, it connects automatically. When the host device enters sleep

mode or is turned off, Orbit Reader reverts to showing stand-alone content, or if connected to a device through the USB port,

reconnects to the screen reader on the other host device.

It is currently not possible to wake up the host from the braille keyboard on the Orbit Reader, like you can with a Bluetooth keyboard.

There are only two ways to wake up a host:

Pressing the Power button

Receiving notification

Before you can take advantage of Bluetooth, you must perform the one-time procedure of pairing Orbit with the host device. With

newer devices, Orbit Reader makes pairing easy by employing a Bluetooth profile named "Just Works." If you use an older host

device that does not support this profile, you need to alter this setting in Orbit Reader's menu.

10/24/2018 20/42

If you set Emulation to RB18, it shows up in the remote device pair list as Refreshabraille#### (where the # sign represents the last

four digits of the serial number of your Orbit Reader).

To pair Orbit Reader to a host device, see the Bluetooth section for the device type.

Connecting iOS with Bluetooth

Connecting Mac with Bluetooth

Connecting Android with Bluetooth

Connecting Windows with Bluetooth

Manage Connections

When you use Bluetooth, it is possible to interrupt Orbit Reader activities with a connected host device. Each time you wake up the

host device, it takes over the Orbit Reader. This behavior makes it very convenient to interact with your host in braille, but if you use

Orbit Reader in Stand-Alone mode or connect it to a screen reader with USB, you may not want the interruption when your phone

wakes up. To return Orbit Reader to what you were doing before the interruption, put the host back to sleep by tapping the Power

button.

To prevent interruptions from occurring altogether, turn off Bluetooth in the Orbit Reader menu or temporarily turn off notifications on

the host device. When you are ready to resume using Bluetooth, either turn it back on from the Orbit Reader menu or use the hotkey

Space + Dots 4 7 from Stand-Alone mode on the Orbit Reader. (You can always return to Stand-Alone mode by pressing Select +

Left Arrow on Orbit Reader.)

Connect to Multiple Hosts

It is possible to pair Orbit Reader with more than one device. For example, you may wish to pair it with both your phone and tablet,

so whichever device you use offers a braille interface.

To more easily use Bluetooth with multiple devices, the screen reader on each of the devices should be set to the same emulation. In

other words, if you have an iPad with a version prior to 10.2, it does not recognize Orbit Reader. Therefore, you should set all

Emulation modes to be compatible with the older version. If all the hosts you are using support Orbit Reader directly, turn Emulation

mode off. If any of them do not support Orbit Reader, set the Emulation mode for all of them to RB18.

While it is possible to pair to a device with Emulation off and later pair to a second device with Emulation on, if you set the emulation

differently between the devices, commands will fail to work correctly. Therefore, it is important to pay particular attention when Orbit

Reader 20 is set to be used with multiple devices. The first device appears to work, but it does not function correctly, because it still

uses Orbit Reader commands instead of Refreshabraille commands, which Orbit expects when Emulation mode is set to RB18.

When connecting to older host devices that do not support Orbit Reader 20, to ensure proper functioning of commands between

Orbit Reader and host devices, set Orbit Reader to emulate RB18.

Note: In Refreshabraille 18 emulation, only the first 18 braille cells are used.

USB Connection Options

Universal Serial Bus (USB) is a technology that makes connecting devices and hosts easy. It provides some advantages over

Bluetooth, because it is both faster and charges your Orbit Reader simultaneously.

Orbit Reader supports three kinds of USB connections (all with the same cable included in the box.)

1. Human Interface Device (recommended)

2. Serial

3. Mass Storage (for turning Orbit Reader's SD card into a drive on your computer)

Human Interface Device (HID)

When using Orbit Reader with a screen reader that supports HID, follow these steps:

1. Turn on Orbit Reader. Orbit reader displays the last braille you were reading.

2. Connect the USB cable to the host and to the Orbit Reader. "-- Charger connected" is displayed.

3. On the Orbit Reader, press Space + Dots 2 7. "-- Remote HID mode" is displayed.

4. Start the screen reader. Orbit responds by displaying what the screen reader is showing. If your screen reader does not support

HID, see Serial below.

To switch back to Stand-Alone mode, press Select + Left Arrow.

10/24/2018 21/42

Serial

When using a screen reader that only supports Serial protocol, follow these steps:

1. Turn on Orbit Reader. It displays the last read material.

2. Connect the USB cable. "-- Charger connected" is displayed.

3. On Orbit Reader, press Space + Dots 3 7. "-- Remote Serial mode" is displayed.

4. Install any necessary drivers. See the Orbit Reader Resources page or your screen reader manufacturer's support webpages.

5. Configure the screen reader. See the pertinent screen reader section in this documentation or consult your screen reader

documentation.

Mass Storage

To use an SD card inserted in the Orbit Reader as a drive on your computer, follow these steps:

1. Turn on Orbit Reader. It responds with the braille from your last activity.

2. Connect the USB cable between Orbit Reader and your computer. "-- Charger connected" is displayed.

3. On the Orbit Reader, press Space + Dots 5 7. "-- Mass Storage mode" is displayed. Depending on your settings, the computer

also responds with a notification of a new drive that is now available.

4. Use your PC to read and write files on the new drive.

To switch back to Stand-Alone mode, press Select + Left Arrow. Do not switch back to Stand-Alone mode while file transfers are in

progress.

Note: If you have hidden files enabled on your device, you may encounter the following files:

".BKP-FILEHISTORY" - This file contains information of the last read position of last 100 files that were opened on the device.

".BKP-RECENTHISTORY" - This file contains information required for software to generate recent file list.

".Logfile" - Log file containing software debug information.

Do not delete these files. You will only see them if you have hidden files enabled.

Buffering Text Input to Remote Devices

If you are a fast typer on the braille display, you may encounter times when the remote device lags behind, slowing down your ability

to input text quickly. Orbit Reader has a Buffer mode that allows you to input text up to 255 characters and send them more quickly to

a remote device, avoiding problems with the speed of input of translated braille.

Note: Buffering text works only when connected with Bluetooth.

To open an input field in the Buffer mode, press Select + Down Arrow. Once open, a cursor indicates that you can start entering text.

Press Dot 8 to send the entire block of text to an open edit box on the remote device. Note that nothing happens when you send text

with no edit box open. Press Select to cancel the action and exit the Buffer mode, without sending any text.

iOS Devices

You can only connect iOS using Bluetooth.

Connecting iOS with Bluetooth

Connecting Orbit Reader to an iOS device provides both braille feedback and the ability to type and control the device with the Orbit

Reader keyboard and directional buttons.

If you are using a version of iOS before 10.2, you must set Orbit Reader to emulate RB18 in the menu.

In default configuration (Just Works), the Orbit Reader is ready to pair with iOS with Bluetooth. Follow these steps to pair with

Bluetooth:

1. Turn on Bluetooth. Bluetooth can be turned on in the Menu or by pressing Space + Dots 4 7 on the Orbit Reader.

2. On the iOS device, go to Settings > General > Accessibility > VoiceOver > Braille.

3. Look for the name of the unit in the list of possible braille displays. It shows up as Orbit Reader or Refreshabraille18 plus the

last four digits of the serial number.

4. Activate the Orbit Reader 20 device in the list to pair.

Confirm code configuration shows a random number on both the Orbit Reader's braille display and on the host device. To confirm the

request, ensure the numbers match and press Dot 8 on the Orbit Reader. Then press the Pair button on your iOS device.

10/24/2018 22/42

For more information about pairing configurations, see Pair (Just Works).

Once you pair the unit, iOS should start sending braille to the Orbit Reader,

and you can use the Orbit Reader input and navigation

keys to control your iOS device.

For more information about iOS and braille displays, see Braille Displays for iOS on the Apple® Accessibility website.

iOS Resources

Apple Blind and Visually Impaired User Community

Andrea's Head Wiki

TechVision Tutorials

Youtube Videos on iPad/iPhone and Braille

Braille Displays for iOS

Common braille commands for VoiceOver navigation using iPhone, iPad, and iPod® touch

iOS Commands

iOS Navigation Commands

Function Keys

Move to previous item Space + Dot 1 or Left Arrow

Move to next item Space + Dot 4 or Right Arrow

Pan braille left Space + Dot 2

Pan braille right Space + Dot 5

Move to the first element Space + Dots 1 2 3

Move to the last element Space + Dots 4 5 6

Scroll right one page Space + Dots 1 3 5

Scroll left one page Space + Dots 2 4 6

Move to the status bar Space + S (Dots 2 3 4)

Select previous rotor setting Space + Dots 2 3

Select next rotor setting Space + Dots 5 6

Move to previous item using rotor setting Space + Dot 3

Move to next item using rotor setting Space + Dot 6

Launch the Task Switcher Space + Dots 1 2 5 twice quickly

Scroll up one page Space + Dots 3 4 5 6

Scroll down one page Space + Dots 1 4 5 6

Go to Notification Center Space + Dots 4 6

Go to Control Center Space + Dots 2 5

iOS Reading Commands

Function Keys

Read all, starting at selected item Space + R

Read all, starting from the top Space + Dots 2 4 5 6

Pause or continue speech Space + P

Announce page number OR number of rows displayed Space + Dots 3 4

iOS General Commands

Function Keys

Activate the Back button if present Space + B (Dots 1 2)

Activate the Delete key Space + D OR Space + Dot 7

Activate the Return key Space + E OR Space + Dot 8

Switch between contracted and uncontracted braille Space + Dots 1 2 4 5

10/24/2018 23/42

Activate the Home button (twice quickly to launch the task

switcher)

Space + H (Dots 1 2 5)

Toggle speech on and off Space + M

Activate the Tab key Space + T (Dots 2 3 4 5)

Context menu Space + Dots 3 5 6

Switch braille input Space + Dots 2 3 6

Volume up Space + Dots 3 4 5

Volume down Space + Dots 1 2 6

Toggle screen curtain on/off Space + Dots 1 2 3 4 5 6

Select all Space + Dots 2 3 5 6

Cut Space + X

Copy Space + C

Paste Space + V

Undo typing Space + Dots 1 3 5 6

Redo typing Space + Dots 2 3 4 6

Activate Eject key Space + Dots 1 4 6

Toggle announcement history Space + Dots 1 3 4 5

Keyboard Help Space + Dots 1 3

iOS iPad Commands

Function Keys

Move to previous container Space + Dots 1 7

Move to next container Space + Dots 4 7

Mac Computers

When connecting Orbit Reader to a Mac, there are two possible methods. One is to connect using the USB cable that came in the

box. The second is to use Bluetooth for a wireless connection.

Connecting Mac with USB

The simplest way to connect the Orbit Reader to a Mac is to connect using a USB Standard-A to Micro-B cable. First, switch Orbit

Reader to the Serial protocol by picking Serial in the USB option in the menu. Next, run VoiceOver by pressing the Apple CMD key +

F5. Then, attach the small end of the USB cable to Orbit Reader 20 with the bumps facing down and the big end to the Mac.

VoiceOver automatically recognizes and starts using the display.

Connecting Mac with Bluetooth

The default configuration for pairing with Mac through Bluetooth is Just Works. If Bluetooth has been turned off, it can be turned on in

the Menu or by pressing Space + Dots 4 7 on the Orbit Reader.

If Orbit Reader is in the default connection, follow these steps to pair the device with a Mac:

1. Turn on Bluetooth. Bluetooth can be turned on in the Menu or by pressing Space + Dots 4 7 on the Orbit Reader.

2. When VoiceOver is on, open VoiceOver Utility by pressing VO-F8.

3. Click the Braille category, click Displays, and then click the Add (+) button.

4. Select Orbit Reader in the list. VoiceOver filters the devices it detects to list only the Bluetooth braille displays that are within

range of your computer and that appear to match a VoiceOver braille display driver.

5. Activate the Orbit Reader 20 device in the list to pair.

Confirm code configuration shows a random number on both the Orbit Reader's braille display and on the host device. Follow steps 1

through 3. Mac shows you a dialog with a security code. To confirm the request, compare the numbers in the Bluetooth dialog on the

Mac with the code shown on your display. If they are the same, press Dot 8 to select Yes in the dialog on the Mac. If the numbers do

not match, press Dot 7 to select No and try the next Orbit Reader in the list. The purpose of this procedure is to allow for more than

one Orbit Reader device to be paired in the same room at the same time without pairing the wrong device.

10/24/2018 24/42

If the Orbit Reader does not work with VoiceOver, it is possible that you are not using the latest Mac OS. If this is the case, you have

two choices:

1. Put the Orbit Reader 20 in Refreshabraille 18 (RB18) Emulation mode. Then connect it as an RB18.

2. Check for updates. APH suggests you update to the latest operating system.

If Orbit Reader is not listed, make sure you have turned on Bluetooth in the Menu.

For more information about pairing configurations, see Pair (Just Works).

Controlling Orbit Reader from Mac

To control how VoiceOver treats the display, follow these steps:

1. Open the VoiceOver Utility by pressing Command + F8 while VoiceOver is running.

2. Scroll down to the Braille option.

3. Select the Braille Display tab.

To turn on contracted braille, check the Contracted Braille box.

For the Mac documentation on connecting and using braille displays, see Apple's VoiceOver Info Guide.

Android Devices

You can only connect Android devices using Bluetooth.

Connecting Android with Bluetooth

In default configuration, the Orbit Reader is ready to pair with Bluetooth. If Bluetooth has been turned off, it can be turned on in the

Menu or by pressing Space + Dots 4 7.

1. Turn on Bluetooth. Bluetooth can be turned on in the Menu or by pressing Space + Dots 4 7 on the Orbit Reader.

2. Turn on the Android device. Android and BrailleBack make a connection sound to indicate successful communication.

3. Check for support.

4. Switch to Emulation mode if necessary.

Currently, to use the Orbit Reader 20, you must change the Emulation mode to Refreshabraille 18 (RB18) in the Menu. Once

Orbit Reader 20 is supported by BrailleBack, it will no longer be necessary to use the Emulation mode.

5. Set up Android device.

To set up your Android device, follow the Install and enable BrailleBack instructions found on the Google™ Android Accessibility

Help webpage.

Input and Output Text With BrailleBack

To input text with your braille keyboard, first enable the keyboard in your Android settings under Language and Input. If you try to use

the keyboard before completing this step, you receive a prompt to change your settings.

To edit a text field, focus and activate the text field. You can then use the Up and Down Arrow keys to move in the text field. When

editing text, BrailleBack switches to eight-dot braille known as Computer Braille. (See 8-Dot Braille Cells section for more

information.) Currently, BrailleBack does not support any other braille as input.

By default, Android is set to display English Computer Braille.

To set BrailleBack to display Unified English Braille (UEB) - grade 2 braille, follow the steps below:

1. Press H to open the Home screen.

2. Arrow to, and select Apps > Settings > Accessibility > BrailleBack > Settings.

3. Arrow through this screen of items to "Braille type" and select.

Android generates a dialog screen which indicates the type of braille being displayed and provides two choices: Literary

braille or Computer Braille.

Computer Braille is checked by default.

4. Arrow to "Literary braille" and select to check this item.

When you check "Literary braille," you are returned to the BrailleBack Settings screen.

5. Arrow to "Literary braille table" and select.

Another dialog screen opens with a selection of output translation tables.

6. Select English (UEB) - grade 2.

You are returned to the BrailleBack Settings screen.

10/24/2018 25/42

BrailleBack Commands

Function Keys

Back Space + B

Home Space + H

BrailleBack help Space + Dots 1 2 3

Notifications Space + N

Recent Apps Space + R

Enter (in Edit field) Space + Dot 8

Backspace (in Edit field) Space + Dot 7

Chromebooks

Currently, you can only use USB to connect to a braille display from a Chromebook™ notebook computer.

Connecting Chromebook with USB

To connect to Chromebook with USB, follow these steps:

1. Press Dots 2 7 + Space to put the Orbit Reader in Remote HID mode.

Alternatively, Press Up Arrow + Select in the menu, and then arrow to USB.

2. Plug the Orbit Reader into the Chromebook USB port.

After a few seconds, the Chromebook detects the device and turns on ChromeVox.

ChromeVox Settings/Commands

To adjust ChromeVox settings or learn commands, go to the Use a braille device with your Chromebook webpage at the

Chromebook Accessibility Help site.

Fire Tablets

To pair the Orbit Reader with VoiceView on your Fire Tablet, follow these steps:

1. Swipe down from the top of the screen with two fingers and double tap on Settings.

2. Find the Accessibility settings and double tap. Then double tap on VoiceView Screen Reader.

3. Navigate to Braille and double tap. Then double tap Pair Bluetooth Braille Display.

4. Make sure your Orbit Reader 20 is on and Bluetooth is on, then double tap on Scan.

5. You should see Orbit Reader 20 followed by the serial number. Double tap on it to pair.

For additional information, go to the Use a Braille Display with Your Fire Tablet webpage on the Amazon Help and Customer Service

site.

Windows PCs

Before connecting Orbit Reader with a PC, you must first decide how you wish to connect: Bluetooth or USB. This is a personal

preference. USB charges the device as you use it, so it is a good choice when your battery is low. Bluetooth does not use wires, so it

is a good choice when you are on the go.

Connecting Windows with USB

If your screen reader does not support HID, switch to the Serial protocol on the Orbit Reader with the hotkey or selecting Serial from

the USB option in the menu.

For Windows versions 7 and newer, the serial interface requires the installation of two drivers: one for the USB and one to make the

USB port appear like a COM port. The only exception is Windows XP. Because it is Serial only, it can just be plugged in. The second

driver is required because many screen reading programs handle braille displays like a serial device. The COM port assignment

driver shows the assigned port number used to communicate with the display. Take a note of the COM port number now for later

use. If you need to look it up at a later time, go to Device Manager while the display is connected and look in the section for Ports:

COM and LPT. One of the COM ports is assigned to Orbit Reader 20. You need to use that number when you set up your screen

reader.

Once the display is connected to the device you wish to use, configure the software to use Orbit Reader.

10/24/2018 26/42

Connecting Windows with Bluetooth

When connecting the Orbit Reader with Bluetooth, you must first choose a pairing configuration.

The default configuration is Just Works. If Orbit Reader is in the default connection, follow these steps to pair the device:

1. On the PC, perform a Bluetooth search.

2. Select the Orbit Reader from the list of Bluetooth devices.

Confirm code configuration shows a random number on both the Orbit Reader's braille display and on the host device. Follow steps 1

and 2; then, to confirm the request, compare the numbers in the Bluetooth dialog on the Windows PC with the code shown on your

display. If they are the same, press Dot 8 to select Yes. If the numbers do not match, press Dot 7 to select No and try the next Orbit

Reader in the list. The purpose of this procedure is to allow for more than one Orbit Reader device to be paired in the same room at

the same time without pairing the wrong device.

The device pairs with the Orbit Reader and an incoming Bluetooth virtual serial port is set up on the PC.

Next, check the Ports list in the Windows Device Manager to find the COM port number assigned to the incoming Bluetooth serial

port. Make a note of this number.

In your screen reader on the PC, set the active Braille display COM port to use as the relevant Bluetooth serial port COM number.

For more information about pairing configurations, see Pair (Just Works).

NonVisual Desktop Access (NVDA)

If you have NVDA (version 2017.1 or later) installed on your PC, it automatically recognizes the Orbit Reader. If NVDA is not

recognizing the display, go to the NVDA Preferences menu and select Braille Settings from the list. From the braille display

drop-down menu, select "Baum/Humanware/APH/Orbit Displays" and click OK.

Note: NVDA works in HID only, not Serial.

If Orbit Reader 20 is not one of the displays shown in the Braille Display list, upgrade NVDA to the newest version and repeat the

process.

NVDA turns on braille output when it is configured for the display. To turn off braille support, select "No Braille" from the Braille

Display options in the Braille Settings menu.

NVDA Commands

Function Key

Move up one line Up Arrow

Move down one line Down Arrow

Move left one character Left Arrow

Move right one character Right Arrow

Enter Select

Job Access With Speech (JAWS)

Currently, the Orbit Reader 20 is supported for JAWS versions 17, 18, and 2018 through a driver. The JAWS driver download and

instructions are available

on the Orbit Research Support webpage.

For JAWS versions older than 17, you must use the RB18 emulation mode on the Orbit Reader.

When connecting Orbit Reader to JAWS by USB, it must be set to use the HID protocol by pressing Space + Dots 2 7.

To connect Orbit Reader by USB follow these steps:

1. Start or restart JAWS

2. Insert + J to bring up JAWS menu

3. Press Enter on Options

4. Down Arrow to Braille and press Enter

5. Tab to Add and press Enter

6. Arrow Up or Down to Aph Refreshabraille 18 and press Space to check the box and select it

10/24/2018 27/42

7. Tab to the Next button and press Enter

8. Select USB

9. Tab to the Next button and press Enter

10. Select Aph Refreshabraille 18 as primary device

11. Tab to the Finish button and press Enter

12. Restart JAWS

Alternate Steps for Bluetooth Connection:

1. Press Space + Dots 4 7

2. Follow steps 1-7 of USB connection

3. Select the COM port where the Orbit Reader 20 is connected (check device manager for COM port or use COM port you saved

from previous steps)

4. Follow steps 9-12 of USB connection

To turn off braille support, follow these steps:

1. Insert + J to bring up JAWS menu

2. Press Enter on Options

3. Down Arrow to Braille and press Enter

4. Up Arrow to No Braille Display

5. Tab to Ok and press Enter

6. Restart JAWS

Note: For a longer description of any of the following commands, turn JAWS keyboard Help on by pressing Insert + 1 and type the

command. Quickly type the command twice to get an even longer description. Press Insert + 1 again to exit JAWS keyboard Help

mode.

Jaws Commands

Jaws Reading Commands

Function Key(s)

Move display to the left Left Arrow

Move display to the right Right Arrow

Move display up one line Up Arrow

Move display down one line Down Arrow

Pan left one braille window Left Advance Bar

Pan right one braille window Right Advance Bar

Top of active window Select + Dots 1 2 3

Bottom of active window Select + Dots 4 5 6

Route braille to active cursor Select + R

Say current line Space + Dots 1 4

Say current word Space + Dots 2 5

Say current character Space + Dots 3 6

Jaws Navigation Commands

Function Keys

Previous document window Space + Dots 1 3

Next document window Space + Dots 4 6

Beginning of file Space + Dots 1 2 3

End of file Space + Dots 4 5 6

Page Up Space + Dots 1 2 3 4 5

Page Down Space + Dots 1 2 4 5 6

Home Space + Dots 1 2 3 4

End Space + Dots 1 4 5 6

Up Arrow Space + Dot 1

10/24/2018 28/42

Down Arrow Space + Dot 4

Previous word Space + Dot 2

Next word Space + Dot 5

Previous character Space + Dot 3

Next character Space + Dot 6

Jaws Editing Commands

Function Keys

Backspace (acts as shift key when typing braille characters) Space + Dot 7

Enter Space + Dot 8

Tab Space + Dots 4 5

Shift + Tab Space + Dots 1 2

Delete current character Space + D

Select all Space + Dots 1 8

Select to top of document Space + Dots 1 2 3 7 8

Select to bottom of document Space + Dots 4 5 6 7 8

Select previous page Space + Dots 1 2 3 4 5 7 8

Select next page Space + Dots 1 2 4 5 6 7 8

Select to beginning of line Space + Dots 1 2 3 4 7 8

Select to end of line Space + Dots 1 4 5 6 7 8

Select previous line Space + Dots 1 7 8

Select next line Space + Dots 4 7 8

Select previous word Space + Dots 2 7 8

Select next word Space + Dots 5 7 8

Select previous character Space + Dots 3 7 8

Select next character Space + Dots 6 7 8

Copy to clipboard Space + Dots 1 4 8

Cut to clipboard Space + Dots 1 3 4 6 8

Paste clipboard Space + Dots 1 2 3 6 8

Undo Space + Dots 1 3 5 6 8

Jaws Windows Commands

Function Keys

Start menu Space + Dots 1 3 4 7 8

Toggle menu bar Space + M

Escape Space + Dots 1 3 5 6

Alt Tab Space + T

Minimize all applications Space + Dots 1 4 5 7 8

Jaws General Commands

Function Keys

Toggle keyboard help Space + Dots 1 4 5 6

Toggle type keys mode Space + Dots 1 2 3 4 5 6

JAWS cursor Select + J

PC cursor Select + P

Route JAWS cursor to PC cursor Select + Dots 1 4

10/24/2018 29/42

Say all to bottom Space + Dots 1 2 3 4 5 6 7 8

Toggle grade 2 translation Space + G (Dots 1 2 4 5)

Toggle grade 2 translation of current word Select + W (Dots 2 4 5 6)

Display last flash message Select + F

JAWS find Space + F

JAWS find next Space + Dots 2 5 7 8

JAWS find previous Space + Dots 2 5 7 8

Toggle active cursor follows braille Select + Dot 1

Toggle braille follows active cursor Select + Dot 4

Cycle through braille modes Select + M

Display six-dot braille Select + Dots 2 3 5

Display eight-dot braille Select + Dots 2 3 6

Toggle cursor shape Select + Dots 1 4 6

Restrict braille cursor Select + R

Toggle characters and attributes Select + Dots 1 6

Braille color marking Select + Dots 1 4

Select attributes to be displayed Select + Dots 1 3

Set 8 characters per space Space + Dots 2 3 6

Set unlimited characters per space Space + Dots 1 3 6

Toggle 8/unlimited characters per space Select + Dots 1 3 4 6

Cycle table reading options Select + T

Cycle table header options Select + H

Graphics labeler Select + G

Adjust JAWS options Select + Dots 1 2 3 6

JAWS window Space + J

Announce time t + Dots 7 and 8

Show script file name Select + Dots 1 2 3 4 5

Run JAWS manager Space + 2 3 7 8

Jaws Special Key Commands

Function Keys

Ctrl + A - Ctrl + Z Dot 8 + any letter

F1 - F10 Dot 7 + Computer Braille number 1 - 0 (0 = F10)

Ctrl + F1 - Ctrl + F10 Dot 8 + Computer Braille number 1 - 0

System Access

To connect Orbit Reader 20 to System Access, the device must be connected by a Standard-A to Micro-B USB cable and set in HID

protocol mode.

1. Press Select + Up Arrow to open Menu

2. Down Arrow to the USB selection

3. Right Arrow to select HID

4. Press Select

5. Press Dot 7 to return to the last location

Another way to quickly switch to HID mode is to use the shortcut keys Space + Dots 2 7.

After running System Access in HID mode, connect the USB cable and wait up to 30 seconds. System Access announces when

Orbit Reader 20 is connected.

10/24/2018 30/42

Dolphin ScreenReader

To connect Orbit Reader 20 to Dolphin ScreenReader, the device must be connected by a Standard-A to Micro-B USB cable and set

in HID protocol mode.

1. Press Select + Up Arrow to open menu

2. Down Arrow to the USB selection

3. Right Arrow to select HID

4. Press Select

5. Press Dot 7 to return to the last location

Dolphin ScreenReader automatically detects Orbit Reader 20 and begins displaying braille.

Another way to quickly switch to HID mode is to use the shortcut keys Space + Dots 2 7.

Window-Eyes

Window-Eyes is no longer being updated. For this reason, there is no Orbit Reader 20 driver for Window-Eyes. To use Window-Eyes

with Orbit Reader, you must emulate Refreshabraille 18.

To connect to Window-Eyes using USB, the Orbit Reader USB protocol must be set to Serial. To do this, follow these steps:

1. Press Select + Up Arrow to open the menu

2. Down Arrow to USB and press Select

3. Press Right or Left Arrow keys to select Serial

4. Press Select

5. To exit, press Dot 7

To select a display, open the Braille Display option in the Window-Eyes File menu. There is no driver for Window-Eyes for versions

prior to 7.1. However, you may use Orbit Reader by selecting any Baum display with input keys. The Vario Connect and the Braille

Connect are two that work. Select the display and set the COM port number established when connecting to a Windows PC.

Narrator

Narrator braille support is in beta. To opt-in to the beta, please follow the instructions found in Chapter 7: Using Narrator with braille

from Microsoft

Support. You will need to have your display in emulation mode as a Refreshabraille 18 and select Baum as the

manufacturer inside the Narrator braille settings.

Key Lock Command

The Key Lock command prevents accidental key presses. The command is to hold Dots 7 8 for two seconds or more. Pressing and

holding these same keys again, unlocks the keys. You can also unlock the keys by turning the device off and back on again.

System Alerts

System alerts, such as "-- Battery low" are indicated by a periodic cycling of the eight dots in the last cell of the device. Any

information in that cell is displayed again after the notification.

Alerts are seen by invoking the Menu or the Editor Context Menu. Pressing Select causes the next alert to appear, if there are any

more. The alert is removed from the alert list once it has been displayed.

If no more alerts are in the list, the first Menu or Editor Context Menu item appears. Pressing Dot 7 clears all pending alerts and puts

you back at the first Menu or Editor Context Menu item.

If the alert message length is longer than 20 characters, you can navigate through the message using the Panning keys. If you are

already in the Menu or Editor Context Menu when an alert happens, you can review the alert by pressing the Space key.

Reboot the Device

If the Orbit Reader suddenly no longer responds to button presses, and you know the battery is not fully discharged, then you may try

rebooting the device by pressing the lower Right Panning key + Dot 8. There is no indication of the reset event. The unit is in power

off condition after reset and needs to be powered on before it can be used.

Device Upgrade Procedure

10/24/2018 31/42

There are two parts to the upgrade procedure: 1) downloading the software and 2) upgrading the Orbit Reader 20. Each part of the

upgrade procedure is explained in the following two sections of this document. Read each section carefully before proceeding.

The following are required to perform the upgrade to the Orbit Reader 20:

Orbit Reader 20 unit

Standard-A to Micro-B USB cable

A PC running Windows XP or later

Orbit Reader 20 Upgrade Utility file found in the Orbit Reader 20 folder (see Download the Software section).

Orbit Reader 20 Release Bin file found in the Orbit Reader 20 folder (see Download the Software section).

For information about bugs, fixes, and additions to the software, see the latest version's Release Notes on the Orbit Research

Firmware Download webpage.

Download the Software

To download a Firmware Upgrade Package for Orbit Reader:

1. Go to the Orbit Research Support webpage.

2. For the most current version, select Download Most Recent Firmware.

Alternatively, to download an older firmware version, select

Download previous firmware release versions, and select the

Upgrade Package of your choice.

3. Select OR20 Firmware Upgrade Package vB0.00.00.XXrYY.

4. Save the zip file to your PC.

When download is complete, follow these steps:

1. Open the folder on your computer where the firmware file was downloaded.

2. Right click on the file and choose "Extract all" OR select the file by arrowing to it, press the application key and choose "Extract

all" from the Context menu.

3. Follow the dialog steps to extract it to a folder of your choice.

When finished, your chosen folder should contain a folder named after the version of the release, for example, OR20_XX.XX.XX.

Make a note of the location of this folder for use in the next section of this document.

Install the Software

To upgrade the firmware, follow these steps:

1. Make sure the Orbit Reader is turned off.

2. Connect the Orbit Reader to the PC using the USB cable.

3. In the Release folder on your PC, run the Orbit Reader 20 Firmware Upgrade Utility. If you receive an error message, see

Troubleshooting section. It shows the message “Do you want to upgrade the device?” in the message field at the bottom of the

dialog.

4. Press and hold down the Right Panning forward button + Dot 5, then press and hold Dot 8. The utility shows the message “Orbit

Reader 20 is connected” in device status field.

5. Release Dot 8 first, then release the Right Panning key and Dot 5. This switches the Orbit reader 20 into Upgrade mode. You

should hear a tone on your PC.

10/24/2018 32/42

6. The utility shows the device serial number and the software version of the device at top right corner of the window. If the serial

number is blank, repeat steps 4 and 5.

7. Activate the Browse button found in the dialog.

8. Browse and select the firmware bin file from your PC. The file is in the folder you unzipped previously.

9. Once file is selected, the dialog shows the software version of the selected file.

10. Activate the Upgrade button.

11. The utility starts upgrading the device.

12. Do not unplug the cable. Wait for the message “Device upgrade has been completed” on the upgrade utility or until the Orbit

Reader reboots and shuts down.

13. The Orbit Reader is upgraded. Turn on to start using the device.

To check for successful upgrade of the device, press Select + Up Arrow to open the Orbit Reader Menu and arrow to "Version" in the

10/24/2018 33/42

list. The version number of the firmware release should match the number on the display.

Language Options (Localization)

In Stand-Alone mode, you can change the operating language to meet your language needs (known as localization). The default

translation table used is English Braille ASCII. The English table is available always within the device and can be used even after

performing localization. Only one other language can be loaded at a time.

To perform localization of the Orbit Reader 20, you can either create and upload the localization files or just upload existing files to

the device. See the Localization Guide for instructions on how to create and upload localization files.

After the localization files are uploaded, you can switch between the files for different language options. See Switch Language for

additional information.

Get Localization Files

To get the Localization files:

1. Go to Orbit Support and download the language of your choice.

2. Go to the saved as file name and extract the zip file to a location that you will remember.

3. Create a folder on the SD card called Locale.

4. Copy the extracted files to the Locale folder on the SD card.

Upload Existing Localization Files

To upload existing localization files:

1. Select Load language from the Localization Preference menu.

2. Select one of three options:

Load Locale - Orbit Reader shows Region code name list for the languages that have both the Table files (.lan) and System

Messages files (.loc) available in the "locale" folder on the SD card, along with the default option. Press Select for the language

file of your choosing.

Load .loc Only - Orbit Reader shows the list of System Messages files, along with the default option. Press Select for the

language file of your choosing. Note: You need to ensure that the correct ASCII to Braille table of the desired region has been

uploaded.

Load .lan Only - Orbit Reader shows the list of Table files, along with the default option. Press Select for the language file of

your choosing.

Switch Localization Files

To switch localization files:

1. Press Up + Select to open the Menu.

2. Arrow down to Switch Language.

3. Right arrow to Switch Locale.

4. Press Select to select a language.

If you only want to switch messages, follow steps 1-3 for switching localization files and press Right Arrow to go to Switch .loc only.

Then, press Select to select from the options.

If you only want to switch the braille language tables, follow steps 1-3 for switching localization files and press Right Arrow twice.

Then press Select to select from the options.

See Switch Language for additional information.

Troubleshooting

If Orbit Reader does not power on or seems to freeze, try these options in the following order:

Option 1: Plug in device.

1. Plug in the unit with the supplied AC wall connector and cable to ensure the battery is charged. This is the most reliable power

source.

2. Let the unit charge for at least an hour or more.

10/24/2018 34/42

3. Hold the Power button for at least two seconds. If the unit is on, it should turn off. If it is off, it should come on.

Option 2: Check SD card.

1. Remove the SD card.

2. Hold the Power button for two seconds.

Option 3: Unplug device.

1. If the unit is plugged in, unplug the power cord.

2. Hold the Power button for at least two seconds.

Option 4: Reset device.

1. Press the Reset command (Dot 8 + right-bottom Panning key). Note: On reset, the unit is unresponsive, and there is no change

to the display.

2. Hold the Power button for at least two seconds.

Option 5: Remove the battery.

Note: Use this option as a last resort.

1. Call APH customer service (800-223-1839) to walk through safely removing the battery.

2. Leave it out for a few minutes.

3. Reconnect it, then try the options again.

If none of the options work and the device does not come on, contact APH Customer Service at cs@aph.org or call 800-223-1839.

Battery Use and Replacement

Orbit Reader includes a rechargeable Lithium-ion battery. It fully charges in about three hours under optimal conditions. A full battery

charge lasts approximately three days of typical use. You can check the Battery status in the Menu.

When the battery in the Orbit Reader gets to 10%, Dot 8 of the 20th cell on the display blinks on and off. This behavior is normal,

indicating that the battery is low.

The battery in Orbit Reader should last several years. Gradually, the battery begins holding its charge for less and less time. When

the amount of time the battery holds a charge becomes inconvenient, it is time for replacement.

To replace the battery, follow these steps:

1. Obtain a replacement battery. Use only the battery designed for Orbit Reader 20. Batteries are available from:

American Printing House for the Blind, Inc.

Orbit Research, Inc.

2. Remove the battery door screws.

3. Remove the battery door.

4. Carefully unplug the connector. Do not pull on the wire. Use the connector instead.

5. Insert the new battery.

6. Replace the cover and screws.

To dispose of batteries properly, call the recycling center at 1-800-822-8837.

Accessories Information

Orbit Reader 20 has two holes on the lower-front corners of the device, one on each corner. The holes are there to provide a means

to attach a strap to the device, with the use of key rings. The key rings can be 10-40 mm in diameter and up to 3 mm in thickness.

The strap and key rings are not included with the purchase of the Orbit Reader 20 from APH. Cases are available for sale at other

retailers, but are not included with the purchase from APH. See the In the Box section for more information.

For additional information about available cases, see the Resources webpage.

10/24/2018 35/42

General Specifications

Dimensions (WxHxD) 6.61 x 1.4 x 4.41 inch (168 x 35.56 x 112 mm)

Weight 0.99 lbs (450 grams)

Braille Cells 20 refreshable eight-dot braille cells; Refresh rate < 0.75 sec

Braille Input Keys 8-key Perkins style keypad, with space bar

High-quality scissor-type keys

Additional Keys Power on/off button

4-way cursor keys and select key

2 panning keys, rocker type

Supported File Formats .txt, .brl, .brf

OS Support Windows: NVDA, JAWS, Window-Eyes, System Access,

Dolphin ScreenReader

Mac OS X: Voiceover

iOS: Voiceover

Android: Brailleback

Chromebook: ChromeVox

Interfaces and Ports 1x Micro-B USB port, USB 2.0, for host interface, firmware

upgrade and battery charging

Bluetooth 2.1, Class 2

Flash Media Slot 1x SD card (4GB to 32GB capacity)

Battery User-replaceable, rechargeable Lithium-Ion battery

Battery Life 3 days of typical use

Charging Method PC charger: Charge over USB Port – 5V, 500 mA Max

Wall charger: Charge over USB Port – 5V, 1000 mA Max; USB

BC1.2 compliant charger required

Environmental Conditions Operational

Temperature: 5° C to 45° C

Humidity: 10% - 70% relative humidity

Storage

Temperature: -15° C to 70° C

Humidity: 5% - 90% relative humidity

Appendices

Appendix A - Computer Braille Table

Computer Braille Table

ASCII Code Character Character Name Dot Combination

33 ! Exclamation 2-3-4-6

34 " Quotation 5

35 # Hashtag/Number 3-4-5-6

36 $ Dollar 1-2-4-6

37 % Percent 1-4-6

10/24/2018 36/42

38 & Ampersand 1-2-3-4-6

39 ' Single Quote 3

40 ( Open Parenthesis 1-2-3-5-6

41 ) Close Parenthesis 2-3-4-5-6

42 * Asterisk 1-6

43 + Plus 3-4-6

44 , Comma 6

45 - Hyphen 3-6

46 . Period/Dot 4-6

47 / Forward Slash 3-4

48 0 Zero (ordinal) 3-5-6

49 1 One (ordinal) 2

50 2 Two (ordinal) 2-3

51 3 Three (ordinal) 2-5

52 4 Four (ordinal) 2-5-6

53 5 Five (ordinal) 2-6

54 6 Six (ordinal) 2-3-5

55 7 Seven (ordinal) 2-3-5-6

56 8 Eight (ordinal) 2-3-6

57 9 Nine (ordinal) 3-5

58 : Colon 1-5-6

59 ; Semicolon 5-6

60 < Less Than 1-2-6

61 = Equal 1-2-3-4-5-6

62 > Greater Than 3-4-5

63 ? Question 1-4-5-6

64 @ Commercial At 4-7

65 A Uppercase A 1-7

66 B Uppercase B 1-2-7

67 C Uppercase C 1-4-7

68 D Uppercase D 1-4-5-7

69 E Uppercase E 1-5-7

70 F Uppercase F 1-2-4-7

71 G Uppercase G 1-2-4-5-7

72 H Uppercase H 1-2-5-7

73 I Uppercase I 2-4-7

74 J Uppercase J 2-4-5-7

75 K Uppercase K 1-3-7

76 L Uppercase L 1-2-3-7

77 M Uppercase M 1-3-4-7

78 N Uppercase N 1-3-4-5-7

79 O Uppercase O 1-3-5-7

80 P Uppercase P 1-2-3-4-7

81 Q Uppercase Q 1-2-3-4-5-7

10/24/2018 37/42

82 R Uppercase R 1-2-3-5-7

83 S Uppercase S 2-3-4-7

84 T Uppercase T 2-3-4-5-7

85 U Uppercase U 1-3-6-7

86 V Uppercase V 1-2-3-6-7

87 W Uppercase W 2-4-5-6-7

88 X Uppercase X 1-3-4-6-7

89 Y Uppercase Y 1-3-4-5-6-7

90 Z Uppercase Z 1-3-5-6-7

91 [ Open Bracket 2-4-6-7

92 \ Back Slash 1-2-5-6-7

93 ] Close Bracket 1-2-4-5-6-7

94 ^ Caret/Circumflex 4-5-7

95 _ Underscore 4-5-6

96 ` Grave Accent 4

97 a Lowercase a 1

98 b Lowercase b 1-2

99 c Lowercase c 1-4

100 d Lowercase d 1-4-5

101 e Lowercase e 1-5

102 f Lowercase f 1-2-4

103 g Lowercase g 1-2-4-5

104 h Lowercase h 1-2-5

105 i Lowercase i 2-4

106 j Lowercase j 2-4-5

107 k Lowercase k 1-3

108 l Lowercase l 1-2-3

109 m Lowercase m 1-3-4

110 n Lowercase n 1-3-4-5

111 o Lowercase o 1-3-5

112 p Lowercase p 1-2-3-4

113 q Lowercase q 1-2-3-4-5

114 r Lowercase r 1-2-3-5

115 s Lowercase s 2-3-4

116 t Lowercase t 2-3-4-5

117 u Lowercase u 1-3-6

118 v Lowercase v 1-2-3-6

119 w Lowercase w 2-4-5-6

120 x Lowercase x 1-3-4-6

121 y Lowercase y 1-3-4-5-6

122 z Lowercase z 1-3-5-6

123 { Open Curly Bracket 2-4-6

124 | Vertical Bar/Line 1-2-5-6

125 } Close Curly Bracket 1-2-4-5-6

126 ~ Tilde 4-5

10/24/2018 38/42

Appendix B - Proper Trademark Notice and Attribution

Adobe® is either a registered trademark or trademark of Adobe Systems Incorporated in the United States and/or other countries.

Android™, Chromebook™, and Google™ are trademarks of Google Inc.

Apple®, iPad®, iPhone®, iPod®, and Mac® are registered trademarks or service marks of Apple Inc., registered in the U.S. and

other countries.

Bluetooth® is a registered trademark of Bluetooth SIG, Inc.

Bookshare® is registered trademarks of Beneficent Technology, Inc.

BrailleBlaster™ and Refreshabraille 18™ are trademarks of the American Printing House for the Blind, Inc.

DAISY, A Better Way to Read is a trademark of the DAISY Consortium.

ScreenReader is a trademark of Dolphin.

iOS® is a registered trademark of Cisco in the U.S. and other countries and is used under license by Apple Inc.

JAWS® is a registered trademark of Freedom Scientific, Inc.

Kindle™ and all related logos are trademarks of Amazon.com, Inc. or its affiliates.

Microsoft® and Windows® are registered trademarks of Microsoft Corporation in the U.S. and/or other countries.

NFB-NEWSLINE® is a registered trademark of the National Federation of the Blind.

Orbit Reader 20™ is a trademark of Orbit Research.

SD is a trademark or registered trademark of SD-3C, LLC in the United States, other countries or both.

Appendix C - Warranty Info

Hardware Limited Warranty

APH's warranty obligations for the Orbit Reader 20 are limited to the terms set forth below:

APH warrants this hardware product against defects in materials and workmanship under normal use for a period of ONE (1) YEAR

from the purchase by the original purchaser ("Warranty Period"). If a hardware defect arises and a valid claim is received by APH

within the Warranty Period, at its option and to the extent permitted by law, APH will either:

1. Repair the product at no charge, using new or refurbished replacement parts (or)

2. Exchange the product with a product that is new or which has been manufactured from new or serviceable used parts and is at

least functionally equivalent to the original product.

A replacement product or part provided by APH or its manufacturer assumes the remaining warranty of the original product or thirty

(30) days from the date of replacement or repair, whichever provides longer coverage for you. When a product or part is exchanged,

any replacement item becomes your property and the replaced item becomes APH's property. Parts provided by APH or its

manufacturer in fulfillment of its warranty obligation must be used in products for which warranty service is claimed.

EXCLUSIONS AND LIMITATIONS

This Limited Warranty applies only to the hardware product manufactured by or for APH that can be identified by the "APH"

trademark, trade name, or logo affixed to it. APH electronic products are also identified by their serial numbers. The Limited Warranty

does not apply to any non-APH hardware product or any software, even if packaged or sold with the APH hardware. Manufacturers,

suppliers, or publishers, other than APH or its manufacturer, may provide their own warranties to the end user purchaser; but APH, in

so far as permitted by law, provides their products "as is." Software distributed by APH or its manufacturer with or without the APH

brand name (including, but not limited to system software) is not covered under this Limited Warranty. Refer to the licensing

agreement accompanying the software for details of your rights with respect to its use. APH does not warrant that the operation of

the product will be uninterrupted or error-free. APH is not responsible for damage arising from failure to follow instructions relating to

the product's use.

This warranty does not apply:

10/24/2018 39/42

1. to damage caused by use with non-APH authorized products

2. to damage caused by accident, abuse, misuse, flood, fire, earthquake, or other external causes

3. to damage caused by operating the product outside the permitted or intended uses described by APH

4. to damage caused by service (including upgrades and expansions) performed by anyone who is not a representative of APH

5. to a product or part that has been modified to alter functionality or capability without the written permission of APH or its

manufacturer

6. if the serial number has been removed or defaced.

TO THE EXTENT PERMITTED BY LAW, THIS WARRANTY AND THE REMEDIES SET FORTH ABOVE ARE EXCLUSIVE AND IN

LIEU OF ALL OTHER WARRANTIES, REMEDIES AND CONDITIONS, WHETHER ORAL OR WRITTEN, STATUTORY, EXPRESS

OR IMPLIED. AS PERMITTED BY APPLICABLE LAW, AMERICAN PRINTING HOUSE FOR THE BLIND SPECIFICALLY

DISCLAIMS ANY AND ALL STATUTORY OR IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE AND WARRANTIES AGAINST HIDDEN OR LATENT

DEFECTS. IF AMERICAN PRINTING HOUSE FOR THE BLIND CANNOT LAWFULLY DISCLAIM STATUTORY OR IMPLIED

WARRANTIES, THEN TO THE EXTENT PERMITTED BY LAW, ALL SUCH WARRANTIES SHALL BE LIMITED IN DURATION TO

THE DURATION OF THE EXPRESS WARRANTY AND TO THE REPAIR OR REPLACEMENT SERVICE AS DETERMINED BY

APH IN ITS SOLE DISCRETION. No American Printing House for the Blind reseller, agent, or employee is authorized to make any

modification, extension, or addition to this warranty. If any term is held to be illegal or unenforceable, the legality or enforceability of

the remaining terms shall not be affected or impaired.

EXCEPT AS PROVIDED IN THIS WARRANTY AND TO THE MAXIMUM EXTENT PERMITTED BY LAW, APH AND ITS

MANUFACTURER IS NOT RESPONSIBLE FOR DIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES

RESULTING FROM ANY BREACH OF WARRANTY OR CONDITION, OR UNDER ANY OTHER LEGAL THEORY, INCLUDING

BUT NOT LIMITED TO LOSS OF USE; LOSS OF REVENUE; LOSS OF ACTUAL OR ANTICIPATED PROFITS (INCLUDING LOSS

OF PROFITS ON CONTRACTS); LOSS OF THE USE OF MONEY; LOSS OF ANTICIPATED SAVINGS; LOSS OF BUSINESS;

LOSS OF OPPORTUNITY; LOSS OF GOODWILL; LOSS OF REPUTATION; LOSS OF, DAMAGE TO, OR CORRUPTION OF

DATA; OR ANY INDIRECT OR CONSEQUENTIAL LOSS OR DAMAGE HOWSOEVER CAUSED INCLUDING THE

REPLACEMENT OF EQUIPMENT AND PROPERTY, ANY COSTS OF RECOVERING, PROGRAMMING OR REPRODUCING ANY

PROGRAM OR DATA STORED IN OR USED WITH THE AMERICAN PRINTING HOUSE FOR THE BLIND PRODUCT, AND ANY

FAILURE TO MAINTAIN THE CONFIDENTIALITY OF DATA STORED ON THE PRODUCT. THE FOREGOING LIMITATION

SHALL NOT APPLY TO DEATH OR PERSONAL INJURY CLAIMS, OR ANY STATUTORY LIABILITY FOR INTENTIONAL AND

GROSS NEGLIGENT ACTS AND/OR OMISSIONS. AMERICAN PRINTING HOUSE FOR THE BLIND DISCLAIMS ANY

REPRESENTATION THAT IT WILL BE ABLE TO REPAIR ANY PRODUCT UNDER THIS WARRANTY OR MAKE A PRODUCT

EXCHANGE WITHOUT RISK TO OR LOSS OF PROGRAMS OR DATA.

CONSUMER PROTECTION LAWS

FOR CONSUMERS WHO ARE COVERED BY CONSUMER PROTECTION LAWS OR REGULATIONS IN THEIR COUNTRY OF

PURCHASE OR, IF DIFFERENT, THEIR COUNTRY OF RESIDENCE, THE BENEFITS CONFERRED BY THIS WARRANTY ARE

IN ADDITION TO ALL RIGHTS AND REMEDIES CONVEYED BY SUCH CONSUMER PROTECTION LAWS AND REGULATIONS.

Some countries, states, and provinces do not allow the exclusion or limitation of incidental or consequential damages or allow

limitations on how long an implied warranty or condition may last, so the above limitations or exclusions may not apply to you. This

warranty gives you specific legal rights, and you may also have other rights that vary by country, state, or province. This Limited

Warranty is governed by and construed under the laws of the country in which the product purchase took place. APH and its

manufacturer, the warrantor under this Limited Warranty, is identified at the end of this document according to the country or region

in which the product purchase took place.

OBTAINING WARRANTY SERVICE

Please access and review the online help resources referred to in the documentation accompanying this hardware product before

seeking warranty service. If the product is still not functioning properly after making use of these resources, please send an e-mail to:

cs@aph.org for instructions on how to obtain warranty service. You must follow APH's warranty procedures.

APH will maintain and use customer information in accordance with the APH Customer Privacy Policy.

If your product is capable of storing software programs, data, and other information, you should make periodic backup copies of the

information contained on the product's hard drive or other storage media to protect the contents and as a precaution against possible

operational failures. Before you deliver your product for warranty service, it is your responsibility to keep a separate backup copy of

the contents, and disable any security passwords. IT IS POSSIBLE THAT THE CONTENTS ON THE DEVICE BEING SERVICED

WILL BE LOST OR REFORMATTED IN THE COURSE OF WARRANTY SERVICE, AND AMERICAN PRINTING HOUSE FOR THE

BLIND AND ITS AGENTS ARE NOT RESPONSIBLE FOR ANY DAMAGE TO OR LOSS OF PROGRAMS, DATA, OR OTHER

INFORMATION CONTAINED ON THE MEDIA OR ANY PART OF THE PRODUCT SERVICED.

Your product will be returned to you configured as originally purchased, subject to applicable updates. You will be responsible for

reinstalling all other software programs, data, and passwords. Recovery and reinstallation of software programs and user data are

not covered under this Limited Warranty.

10/24/2018 40/42

Return Policy

If you are not satisfied with your product, please call (800) 223-1839 for a Return Material Authorization (RMA) request within 30

calendar days of the receipt of the product. The product must be returned to APH within 15 calendar days of the issuance of the

RMA. All products must be in satisfactory condition packed in the original, unmarked packaging including any accessories, manuals,

documentation, and registration that shipped with the product. Less than satisfactory condition includes but is not limited to scratched

product, damaged packaging, and missing accessories (power adaptors, CDs, manuals, etc.). Items received in non-resalable

condition will not be accepted and will be returned. APH may refuse a product return if the Return Authorization Number is not written

on the outside of the shipping container.

Shipping and Handling

Shipping and handling charges apply to destinations in the continental United States, Alaska, and Canada. All shipments are sent by

surface carrier, unless otherwise stated. Please call APH customer service at (800) 223-1839 for shipment by more expedient

means. Extra charges will apply. APH does not accept CODs. Please note:

All prices, charges, and specifications are subject to change without notice. All prices are in U.S. dollars, F.O.B. APH. APH accepts

VISA, MasterCard, American Express, and Discover Charge Cards. Sales tax is additional, where required by law.

In case of trouble operating your device, please contact the manufacturer.

American Printing House for the Blind, Inc.

1839 Frankfort Ave.

PO Box 6085

Louisville, KY 40206

Phone: 800-223-1839

E-mail: cs@aph.org

Web: www.aph.org

Appendix D - FCC Information

FCC Notice

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC rules.

These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment

generates uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause

harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular

installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the

equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

• Reorient or relocate the receiving antenna.

• Increase the separation between the equipment and the receiver.

• Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

• Consult the dealer or an experienced radio/TV technician for help.

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

(1) This device may not cause harmful interference, and

(2) This device must accept any interference received, including interference that may cause undesired operation.

FCC Caution

To ensure continued compliance follow the installation instructions and this device must be installed with a computer or peripheral

device certified to comply with Part 15 of the FCC rules for the Class B limits. Use shielded interface cables only. Use the power

adapter that is supplied with the device to charge its internal battery pack.

Any changes or modifications not expressly approved by the party responsible for compliance could void the user’s authority to

operate this equipment.

Files

Back_edge.jpg 45.2 KB 11/21/2016 LBlakey

braille-cells.jpg 66.5 KB 11/21/2016 LBlakey

OR20-3keys.jpg 30.2 KB 11/21/2016 LBlakey

OR20-angle.jpg 139 KB 11/21/2016 LBlakey

OR20-cells.jpg 59 KB 11/21/2016 LBlakey

OR20-Angle2.jpg 113 KB 11/21/2016 LBlakey

OR20-face.jpg 157 KB 11/21/2016 LBlakey

OR20-Flat.jpg 118 KB 11/21/2016 LBlakey

10/24/2018 41/42

OR20-inputkeys.jpg 59.5 KB 11/21/2016 LBlakey

OR20-keyrings.jpg 51.7 KB 11/21/2016 LBlakey

three-keys.jpg 31.3 KB 11/21/2016 LBlakey

top-row.jpg 49.4 KB 11/21/2016 LBlakey

UpgradeUtility-Figure1.jpg 31.4 KB 11/21/2016 LBlakey

UpgradeUtility-Figure2.jpg 40.3 KB 11/21/2016 LBlakey

UpgradeUtility-Figure3.jpg 38.7 KB 11/21/2016 LBlakey

UpgradeUtility-Figure4.jpg 39.1 KB 11/21/2016 LBlakey

OR20-Ports.jpg 64.4 KB 11/21/2016 LBlakey

Powered by TCPDF (www.tcpdf.org)

10/24/2018 42/42